| Part PN | Part Name |
| --- | --- |
| 120B38180S | IC, Analog, DAC, FMS3818, 3x8bit Video,Ms/S |
| 116B90000S | IC, Periph., DM9000, Fast Ethernet Controller, Local bus, 3.3V |
| 188C01332 | PCB, EB-X270 Rev 1.2 |
| 182K14000S | Conn., B2B, 2x70 cont., P:0.6mm, Rec, H:4mm |
| 106RV2450S | IC, Logic, 74VCX163245, 16bit Dual-supp xcvr w/Config volt translation |
| 123V16000S | IC, Volt Reg., LM2743, Sync Buck controller, Step down |
| 182K00802 | Conn., RJ45, W/ 100B-T transformer & term res & 2KV cap |
| 111H42030I | IC, Memory, FLASH, Ind., 29LV160, 2Mx8/1Mx16, 120nS, 3.3V |
| 116B95550S | IC, Periph, 16-bit I2C and SMBus GPIO I/O expander |
| 123M3R000S | IC, Volt. Reg., Dual LDO, 3/3V 150/300mA |
| 141B25600I | XTAL., Industrial, 25.000MHZ, 50PPM, 7x5mm SMT, -40°C+85°C |
| 147A2R201S | Inductor, 2.2uH, Isat=14A, 18Mohm DCR |
| 182K00304S | Conn. DC Jack, 3.9/1.3mm ext/int Dia., SMT, 3A |
| 151B10780S | Cap., 100uF, 6.3V, X5R, 20%, 1210, H=2.5mm |
| 151C47680S | Cap., 47uF, 16V, X5R, 20%, 1210, H=2.5mm |
| 124S82200S | IC, Analog switch, RYC8220-1M, Dual USB current limited switch, so-8 |
| 190C02000S | Conn., Header, 2x10, 100MIL, SMT, DR20, 6mm mating length |
| 130E83500S | Diode, Schottky, MBRD-835L, 8A, 35V, DPAK |
| 133A99260S | Transistor Array, Dual N-MOSFET, 2.5V Vgs,6A, 20V |
| 180K01501 | Conn., DB15 HD, 3.08mm, RA, Fem, Blue, PC99, TH |
| 123V90000S | IC, Volt. Reg. Si9183DT-AD-T1, Adjustable, 150mA |
| 106C20050S | IC, Logic, MicroGate, NC7SZ05P5, Open drain inverter, SC70 |
| 180K00400S | Conn., USB Type A, FEM, RA, PC Mount, SMT |
| 132D42200S | Diode, TVS, Dual USB 2.0 ESD protection, Low cap (1pF) |
| 151J10250S | Cap., 1nF, 2KV,SMD 1812, X7R, 10%, 7"REEL |
| 132D10010S | Diode, Transzorb, 5.8V, Bi-Di, 600W |
| 139A10140S | LED, Chip 3x1mm, R/A, Orange, 3.2mcd, 20mA, 2.2V fwd |
| 170B00001S | SWITCH, Push-B,Tact, 6x6 mm, 4.3mm High, SMT |
| 106C20000S | IC, Logic, MicroGate, NC7SZ00P5, 2-input NAND gate, SC70 |
| 383D52530 | Spacer, Hexagonal, M2x.4 thread, 4.14mm Length, Brass with tin coating |
| 139A10130S | LED, Chip 3x1mm, R/A, Green, 16mcd, 20mA, 2.1V fwd |
| 147D25170S | Ferrite Bead, 2518121217Y1,1812 SMD, 1.5A, (z)120ohm, (dcr) 0.05ohm |
| 106C20320S | IC, Logic, MicroGate, NC7SZ32, 2 Input OR gate, SC70 |
| 130E05402S | Diode, Schottky, sgl, BAT54WS, SOD-323 |
| 151B10620S | Cap., 10uF, 6.3V, X5R/X7R, 20%, 0805 |
| 380J52020 | Screw, M2, Pan head, Philips, 3.5mm +-0.1mm Length, stainless Steel, Nickel or Zinc plating |
| 151B47510S | Cap., 4.7uF, 6.3V, X7R or X5R, 10-20%, 0603 |
| 190C00200 | Conn., Header, 1x 2, 100MIL, TH, JP2, 3mm tail, 6mm mating length |
| 190C00400 | Conn., Header, 1x4, 100MIL, TH, JP4, 3mm tail, 6mm mating length |
| 147D60100S | Ferrite Bead, 0603, Z=330 Ohm@100Mhz,200mA, 0.5 Ohm DCR |
| 151B10511S | Cap., 1uF, 6.3V, X5R, 10%, 0603 |
| 185B10010 | Conn., Jumper Plug (shunt), 2.54mm (.1") spacing, Gold plated, Black color |
| 163K04102S | Res., Array, 1K, 8P4R, 5%, 0805 |
| 163K04510S | Res., Array, 51R, 8P4R, 5%, 0805 |
| 163K04750S | Res., Array, 75R, 8P4R, 5%, 0805 |
| 151D10100S | Cap., 100pF, 0402, NPO, 50V, 5% or 1% |
| 151D22000S | Cap., 22pF, 50V, NPO, 5%, 0402 |
| 151D22200S | Cap., 2.2nF, 50V, X7R, 10%, 0402 |
| 151D10200S | Cap., 1nF 0402, X7R, 50V, 10% |
| 151B10400S | Cap., 100nF, 10V, X7R or X5R, 10%, 0402 |
| 161C22121S | Res., 22.1K, 1%, 0402, 1/16W |
| 151C10300S | Cap., 10nF, 25V, X7R, 10%, 0402 |
| 161C34801S | Res., 348R, 1%, 0402, 1/16W |
| 161C47501S | Res., 475R, 1%, 0402, 1/16W |
| 161C75011S | Res., 7.5K, 1%, 0402, 1/16W |
| 161C10021S | Res., 10K, 1%, 0402, 1/16W |
| 161D10055S | Res., 10M, 5%, 0603 |
| 161C30021S | Res., 30K, 1%, 0402, 1/16W |
| 161C47511S | Res., 4.75K, 1%, 0402, 1/16W |
| 161C56211S | Res., 5.62K, 1%, 0402 |
| 161C68111S | Res., 6.81K, 1%, 0402, 1/16W |
| 161C71521S | Res., 71.5K, 1%, 0402, 1/16W |
| 163K04103S | Res., Array, 10K, 8P4R, 5%, 0805 |
| 161C10035S | Res., 100K, 5%, 0402, 1/16W |
| 161C22005S | Res., 220R, 5%, 0402, 1/16W |
| 161C10R05S | Res., 10R, 5%, 0402 |
| 161C47025S | Res., 47K, 5%, 0402, 1/16W |
| 161C00005S | Res., 0R, 5%, 0402, 1/16W |