SB-X270

Assembling Instructions

CompuLab

# Revision Notes

| 24-Dec-2005 | - First version |
| --- | --- |
| 05-Feb-2006 | - Added note regarding reference of spacers. |
| 16-Apr-06 | Rev 1.1 update: - New components in BOM: Q10, RN11, RN27, R66, D9, R2, R65 - Removed from BOM: R18, C138, C60, C61, C62 - Changed footprint: C42, C46, C51, U11, C117, C123 - Added U30 bypass (RN11, RN27) - R48 changed to 161C22121S. - R40, R49 changed to 161C14031S. |
| 27-Jun-06 | - Added a note regarding standoff assembly. - R40 changed to 161C71521S. - R50 changed to 161HR0121S. |
| 1-Jul-2006 | - Altered jumper options - R32 value changes to 4.99K/1% (161C49911S) |
| 17-Sep-06 | - Removed C138 from assembly instructions. - Altered Fx option comments. - Fixed C2 option screw numbering. |
| 20-Sep-06 | - Fixed C42,C46,C51 P/N in assm. Instructions. - Fixed screw numbering in Cx option. - Added references of mechanical components to BOM: P32, P33, JP1-JP4, LBL2, PCB1. - Added LBL1 to BOM. - Option X2: X11 changed to X1; fixed both assembly instructions and BOM. |
| 21-Sep-06 | - Added R3 to BOM. |
| 26-Sep-06 | - Fixed the comment regarding RN polarity. |
| 10-Oct-06 | - Added note about cutting P25, P26 (RJ45 connectors) pins from PS before soldering (Options F and E) |
| 1-Feb-07 | - Removed C140,C143,C150,C151 from BOM. |
| 11-Apr-07 | Updated BOM for rev 1.2. Bullets below reflect the changes relevant to this document only. - All 100uF caps were switched to 151B10780S (1210 ceramic). - Added components to F1 option: R71, R40,RN41, P36, P35. - P19 changed to 190C01402 (2x7 100mil). - Added P20 to L option. - Added Q12 for “Sx not assembled” option. - C93, C94 changed to 151K33000S (33pF). - Removed C95, L7. |
| 23-Oct-07 | Updated BOM for rev 1.3 - Removed V option. - L9 replaced with Delta MPL73. - Routed USB3 to mini-USB connector (RN42) |

# Components which should not be assembled

## [Fx]  - [P] - [R] - [Ex] - [L] - [M] - [Cx] - [Xx] - [Yx] - [Sx]

"F" option controls assembling of the front panel. “Fx” can be omitted or can have values “F0” or “F1”.

If "F1" is not specified, then don't assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| U25 | 180K00801 | 1 | USB_STACKED | Dual USB-A connector |
| C98, C99, C136, C137, C139 | 151B10400S | 5 | 100nF | USB supply bypass, RJ45 TDC/RDC filtering |
| P24 | 180K01501 | 1 | DB15 | VGA connector |
| C100, C101, C102, C103, C104 | 151D10000S | 5 | 10pF | EMI filtering |
| P25, P26 | 182K00802 | 2 | RJ45 MODULE | RJ45 connector |
| R35 | 161C00005S | 1 | 0R | RJ45 TDC to VCC bypass |
| DS2, DS3 | 139A10140S | 2 | LED-ORG R/A | Ethernet status LED |
| DS4, DS5 | 139A10130S | 2 | LED_GREEN R/A | Ethernet status LED |
| R36, R37 | 161C22005S | 2 | 220R | Ethernet LED PU |
| P19 | 190C01402 | 1 | 2x7THHDR | JTAG header |
| R23 | 161C00005S | 1 | 0R | JTAG VCC3.3 short |
| P27 | 180K00600 | 1 | RJ11 | COM1 connector |
| R71 | 161C00005S | 1 | 0R | MiniUSB power rail |
| RN40 | 163K04000S | 1 | 0R, 8P4R | USB routing to front panel |
| RN42 | 163K04000S | 1 | 0R, 8P4R | USB routing to miniUSB |
| P36 | 180K00901S | 1 | MINI-USB | MiniUSB socket |
| P35 | 190C00600 | 1 | 1x6HDR | A/D header |

If "F1" is specified, then don't assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| RN38 | 163K04000S | 1 | OR, 8P4R | USB routing to 100mil header |

If "Fx" is not specified, then after assembling front panel area should be cut from PCB.

Important: don't cut front panel PCB if "Fx" option is specified, although v-groove cut-line exists.

P26 and related components (C139,DS3,DS5,R37)   assembling is also controlled by E2 option.

P24 and related components (C100,C101,C102,C103,C104) assembling is also controlled by M option.

If P25,P26 are assembled, their pins from PS should be cut before soldering.

## [Fx] -  [P]  - [R] - [Ex] - [L] - [M] - [Cx] - [Xx] - [Yx] - [Sx]

"P" option controls assembling of the PC104 connector and related components.

If "P" is not specified, then don't assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| P4 | 182N10400 | 1 | PC104MOD | PC104 COMBO 32x2 + 20x2 stacktrough |
| C3 | 151B10400S | 1 | 100nF | Bypass capacitor |
| C4 | 151C10300S | 1 | 10nF | Bypass capacitor |
| C12 | 151B10620S | 1 | 10uF | Bypass capacitor |
| RN2 | 163I08822S | 1 | 8.2K/10P8R | Pulldowns |
| P4b | 203G00640 | 1 | -- | Guide pins for PC104 |
| P4a | 203G00400 | 1 | -- | Guide pins for PC104 |

## [Fx] - [P] -  [R]  - [Ex] - [L] - [M] - [Cx] - [Xx] - [Yx] - [Sx]

"R" option controls assembling of the PC104+  connector and related components.

If "R" is not specified, then don't assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| P5 | 190D12001 | 1 | PC104 Plus | PC104+ connector |
| C1 | 151B10620S | 1 | 10uF | Bypass capacitor |
| C2, C13 | 151B10400S | 2 | 100nF | Bypass capacitors |
| RN15 | 163K04330S | 1 | 33R/8P4R | PCI clock terminations |
| RN3 | 163E04151S | 1 | 150R/8P4R | IDSEL |

P5 sometimes has a bulge, as shown below:

While P5 assembled, the bulge orientation should be towards card's PCB edge.

## [Fx] - [P] - [R] -  [Ex]  - [L] - [M] - [Cx] - [Xx] - [Yx] - [Sx]

"Ex" can take values of "E1" or "E2".  The "E1" option has no meaning, it is used only for clarity. Don't take any actions regarding "E1" option. (Header of first Ethernet port is assembled always)

"E2" option controls assembling of second port Ethernet components.  If "E2" is not specified, then don't assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| U10 | 116B81390S | 1 | RTL8139DL | Ethernet controller |
| R9, R10 | 161C10001S | 2 | 100R, 1% | Termination resistors |
| R17 | 161C20001S | 1 | 220R | IDSEL resistor |
| R11 | 161D56211S | 1 | 5.62K, 1% | RTSET resistor |
| L3, L4, L5 | 147D60100S | 3 | EMI 0603-330 | Filters |
| X1 | 141B25600I | 1 | 25.000MHz | Xtal |
| Q1 | 131A11970S | 1 | 2SB1197K | Ethernet low voltage transistor |
| C36, C37 | 151D22000S | 2 | 22pF | Xtal caps |
| C42, C46, C51 | 151B10400S | 3 | 100nF | VCC3.3 bypass Analog filters |
| C40, C41 | 151B10400S | 2 | 100nF | 2.5v bypass |
| C39 | 151B10620S | 1 | 10uF | Low voltage filter |
| P26 | 182K00802 | 1 | RJ45 MODULE | RJ45 connector |
| C139 | 151B10400S | 1 | 100nF | RJ45 TDC/RDC filtering |
| DS3 | 139A10140S | 1 | LED-ORG R/A | Ethernet status LED |
| DS5 | 139A10130S | 1 | LED_GREEN R/A | Ethernet status LED |
| R37 | 161C22005S | 1 | 220R | Ethernet LED PU |
| U11 | 111S40600S | 1 | 93C46B/DFN | Serial EEPROM |

If “E2” is specified, then don’t assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| RN14 | 163K04000S | 1 | 0R/8P4R | REQ/GNT bypass |

If P26 is assembled, it’s pins from PS should be cut before soldering.

## [Fx] - [P] - [R] - [Ex] -  [L]  - [M] - [Cx] - [Xx] - [Yx] - [Sx]

"L" option controls assembling of components for LCD panel interface.  If "L" is not specified, then don't assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| Q2 | 131D76040S | 1 | IRF7604 | Voltage key |
| Q8, Q7 | 131D03350S | 2 | NDS335N | LCD voltage control |
| P13 | 182P05100S | 1 | FPC 0.3mm | LCD Connector |
| RN18, RN19, RN20, RN21, RN22, RN23, RN24 | 163K04330S | 7 | 33R/8P4R | Terminations |
| R1 | 161C47511S | 1 | 4.75K/1% | Pullup |
| C57, C58 | 151C10300S | 2 | 10nF | LCD bypass |
| C59,C60, C61,C62 | 151B10400S | 4 | 100nF | LCD bypass |
| C56 | 151B10620S | 1 | 10uF | Filtering cap |
| P20 | 182P04000S | 1 | FPC 40 0.5mm | LCD connector |

## [Fx] - [P] - [R] - [Ex] - [L] -  [M]  - [Cx] - [Xx] - [Yx] - [Sx]

"M" option controls assembling of components for CRT interface. If "M" is not specified, then don't assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| U13 | 120B38180S | 1 | FSM3818 | Video DAC |
| RN17 | 163K04750S | 1 | 75R/8P4R | Termination |
| R19 | 161C34801S | 1 | 348R | Reference resistor |
| C52, C55 | 151B10400S | 2 | 100nF | Reference voltage filter Compensation capacitor |
| C53 | 151B10620S | 1 | 10uF | Filter |
| P24 | 180K01501 | 1 | DB15 | VGA connector |
| C100, C101, C102, C103, C104 | 151D10000S | 5 | 10pF | EMI filtering |

## [Fx] - [P] - [R] - [Ex] - [L] - [M] -  [Cx]  - [Xx] - [Yx] - [Sx]

“Cx” option controls assembling of  PCMCIA & CardBus glue logic and connectors.

If “Cx” is not specified, then do not assemble the following components:

| Ref | P/N | Qty | Type | Function |
| --- | --- | --- | --- | --- |
| P6, P7 | 182K10000S | 2 | BERG 73277 | PCMCIA connector receptors |
| P9 | 182E10011S | 1 | CARDBUS_SKT_UPR | Upper CardBus socket |
| P10 | 182E10010S | 1 | CARDBUS_SKT | Lower PCMCIA socket |
| U8 | 102E80660S | 1 | FSA266 | PCI FRAME# key |
| C130 | 151B10780S | 1 | 100uF | Power cap |
| SCREW 5..8 | 380J52080 | 4 | M2, 10mm | connector screws |
| SCREW 1..4 | 380J52110 | 4 | M2, 16mm | connector screws |
| NUT1..4 | 381A52000 | 4 | M2, Hex, Steel | Nuts for socket screws |

If “C2” is not specified, then do not assemble the following components:

| Ref | P/N | Qty | Type | Function |
| --- | --- | --- | --- | --- |
| P7 | 182K10000S | 1 | BERG 73277 | CardBus connector receptor |
| P9 | 182E10011S | 1 | CARDBUS_SKT_UPR | Upper CardBus socket |
| U8 | 102E80660S | 1 | FSA266 | PCI FRAME# key |
| SCREW 1..4 | 380J52110 | 4 | M2, 16mm | connector screws |

If “C2” is specified, then do not assemble the following components:

| Ref | P/N | Qty | Type | Function |
| --- | --- | --- | --- | --- |
| SCREW 5..8 | 380J52080 | 4 | M2, 10mm | connector screws |

## [Fx] - [P] - [R] - [Ex] - [L] - [M] - [Cx] -  [Xx]  - [Yx] - [Sx]

“X2” option controls COM2 & COM4 serial drivers assembling. If “X2” is not specified then do not assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| U30 | 116A32430I | 1 | MAX3243 | RS232 driver |
| C105, C109 C131, C135 | 151B10400S | 4 | 100nF | Charge-pump caps Power filters |

If “X2” is specified then do not assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| RN1, RN27 | 163K04000S | 2 | 0R | Driver bypass |

## [Fx] - [P] - [R] - [Ex] - [L] - [M] - [Cx] - [Xx] -  [Yx]  - [Sx]

“Yx” option controls COM3 serial drivers assembling. If “Yx” is not specified then do not assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| U20 | 116A32430I | 1 | MAX3243 | RS232 driver |
| U21 | 116A34910S | 1 | MAX3491E | RS422 driver |
| C75, C76 C78, C79 | 151B10400S | 4 | 100nF | Charge-pump caps Power filters |
| R27, R28 | 161D12001S | 2 | 120R/1% | RS422 terminations |
| R26 | 161C15011S | 1 | 1.5K/1% | mix |

If “Y2” is specified then do not assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| U21 | 116A34910S | 1 | MAX3491E | RS422 driver |
| R27, R28 | 161D12001S | 2 | 120R/1% | RS422 terminations |
| RN37, RN25 | 163K04000S | 2 | 0R/8P4R | Bypass resistors |

If “Y4” is specified then do not assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| U20 | 116A32430I | 1 | MAX3243 | RS232 driver |
| C75, C76 C78, C79 | 151B10400S | 4 | 100nF | Charge-pump caps Power filters |
| R26 | 161C15011S | 1 | 1.5K/1% | mix |

If “Y6” is specified then do not assemble:

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| RN37, RN25 | 163K04000S | 2 | 0R/8P4R | Bypass resistors |

## [Fx] - [P] - [R] - [Ex] - [L] - [M] - [Cx] - [Xx] - [Yx] -  [Sx] ]

“Sx” option controls power supply circuitly assembly.

If  “Sx” option is not specified, then do not assemble

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| U26 | 123V6R000I | 1 | TPS40061 | 48 switching controller |
| R40 R41 | 161C71521S | 2 | 71.5K | Feed forward resistor Current sense |
| R44 R49 | 161C14031S | 2 | 140K | Switching freq. resistor 48V Feedback divider |
| R45 R56 | 161C10021S | 2 | 10K | Sync pull-down 3.3V feedback divider |
| R48 | 161C22121S | 1 | 22.1K/1% | 48V Feedback divider |
| C121 | 151C22310S | 1 | 22nF | Soft start capacitor |
| C127 | 151D20110S | 1 | 200pF | 48V Feedback compensation |
| C128 | 151C10300S | 1 | 10nF | 48V Feedback compensation |
| C129 | 151K47210S | 1 | 4.7nF | 48V Feedback compensation |
| R47 | 161C49901S | 1 | 499R | 48V Feedback compensation |
| R46 | 161C47511S | 1 | 4.75K/1% | 48V Feedback compensation |
| C110, C111, C112, C113, C132 | 151B10620S | 5 | 10uF | 5V bypass caps |
| C114, C142 | 151B10780S | 2 | 100uF | 5V bypass caps |
| C124, C125 | 151B10780S | 2 | 100uF | 48V switcher Co |
| C126 | 151B10511S | 1 | 1uF | 48V switcher Co |
| C120 | 151B10400S | 1 | 100nF | 5V reference bypass |
| C117, C123 | 151K10410S | 2 | 100nF | -10V reference bypass 10V reference bypass |
| C118 | 151D10100S | 1 | 100pF | Current sense filter |
| R43 | 161C10R05S | 1 | 10R | Switch sense resistor |
| C108, C115, C116 | 151E47560S | 3 | 4.7uF | Input capacitor |
| Q3, Q9, Q5 | 131D74150I | 3 | Si7415DN | Upper switch |
| Q4 | 131D78500I | 1 | Si7859DP | Lower switch |
| L8 | 147A15001S | 1 | 15uH | 48V switcher Lo |
| U31 | 132D56000S | 1 | P6SMB56C | Transzorber |
| U28 | 123V6R500S | 1 | TPS64202 | 3.3V switching controller |
| D2 | 130E83500S | 1 | MBRD-835L | Low side rectifier |
| L9 | 147A2R201S | 1 | 2.2uH | 3.3V Lo |
| R50 | 161HR0121S | 1 | 12mR | 3.3V current sense |
| D8 | 130E05402S | 1 | BAT54WS | 5V protection |
| R55 | 161D17421S | 1 | 17.4K | 3.3V feedback divider |
| C134 | 151D10000S | 1 | 10pF | 3.3V feedback filter |

If  “Sx” option is specified, then do not assemble

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| Q12 | 131D76040S | 1 | IRF7604 | VCC3_3 to VCC3_3SBY switch |

If  “S5” is specified, then do not assemble

| Ref | P/N | Qty | Value | Function |
| --- | --- | --- | --- | --- |
| U26 | 123V6R000I | 1 | TPS40061 | 48 switching controller |
| R40 R41 | 161C71521S | 2 | 71.5K | Feed forward resistor Current sense |
| R44 R49 | 161C14031S | 2 | 140K | Switching freq. resistor 48V Feedback divider |
| R45 | 161C10021S | 1 | 10K | Sync pull-down |
| R48 | 161C22121S | 1 | 22.1K/1% | 48V Feedback divider |
| C121 | 151C22310S | 1 | 22nF | Soft start capacitor |
| C127 | 151D20110S | 1 | 200pF | 48V Feedback compensation |
| C128 | 151C10300S | 1 | 10nF | 48V Feedback compensation |
| C129 | 151K47210S | 1 | 4.7nF | 48V Feedback compensation |
| R47 | 161C49901S | 1 | 499R | 48V Feedback compensation |
| R46 | 161C47511S | 1 | 4.75K/1% | 48V Feedback compensation |
| C124, C125 | 154B107D1S | 2 | 100uF | 48V switcher Co |
| C126 | 151B10511S | 1 | 1uF | 48V switcher Co |
| C120 | 151B10400S | 1 | 100nF | 5V reference bypass |
| C117, C123 | 151K10410S | 2 | 100nF | -10V reference bypass 10V reference bypass |
| C118 | 151D10100S | 1 | 100pF | Current sense filter |
| R43 | 161C10R05S | 1 | 10R | Switch sense resistor |
| C108, C115, C116 | 151E47560S | 3 | 4.7uF | Input capacitor |
| Q3, Q9 | 131D74150I | 2 | Si7415DN | Upper switch |
| Q4 | 131D78500I | 1 | Si7859DP | Lower switch |
| L8 | 147A15001S | 1 | 15uH | 48V switcher Lo |
| U31 | 132D56000S | 1 | P6SMB56C | Transzorber |

# Notes about assembling process

- Remove pick-up clips from all connectors.

- X1 & X2 don’t have polarity and can be assembled in both possible orientations.

- Put LBL2 (round label P/N 374R16P16) beneath the battery BT1, to prevent its body from contact with PCB."

- All RNs do not have polarity.

- P6 and P7 should be assembled according to leading pins.

- Cut pin 20 of P8.

- Assemble screw PN 380J52080 in the holes marked screw21, screw 22, screw23 and screw24 from PS and secure with spacer PN 383D52530 from the CS, in ref SP3, MCH2, MCH3, MCH4.

- Shunts: use JP1..JP4 (Shunt jumper PN 185B10010.)

| Header name | Pins to short |
| --- | --- |
| P15 | 2-3 |
| P15 | Pin 11 only |
| E2 | 2-3 |
| E4 | 2-3 |

Also see the "Common Part of Assembling Instruction" document. Don't perform assembling before reading that document !

# After assembling

- Gently open the FPC connector P13, to ensure that it is not stucked. It can then be left open.