Part Number Description

| Part PN | PartName: |
| --- | --- |
| 120B38180S | IC, Analog, DAC, FMS3818, 3x8bit Video,Ms/S |
| 161C49901S | Res., 499R, 1%, 0402, 1/16W |
| 151B10400S | Cap., 100nF, 10V, X7R or X5R, 10%, 0402 |
| 151C10300S | Cap., 10nF, 25V, X7R, 10%, 0402 |
| 151D20110S | Cap., 200pF 0603, NPO, 50V, 0603 |
| 161C00005S | Res., 0R, 5%, 0402, 1/16W |
| 161C10021S | Res., 10K, 1%, 0402, 1/16W |
| 190C01402 | Conn., Header, 2x 7, 100MIL, TH |
| 161C22005S | Res., 220R, 5%, 0402, 1/16W |
| 161C10001S | Res., 100R, 1%, 0402, 1/16W |
| 180K00600 | Conn., RJ11,Phone Jack, 6P6C, RA, TH |
| 106E32451S | IC, Logic, 74LCX245,3.3V, TSSOP-20,170MIL (4.4mm) |
| 190C05000S | Conn., Header, 2x25, 100MIL, SMT, 6mm mating length |
| 180K00801 | Conn., USB x2 (dual) Type A, FEM, RA, PC Mount, TH |
| 161C49911S | Res., 4.99K, 1%, 0402, 1/16W |
| 161C33211S | Res., 3.32K, 1%, 0402, 1/16W |
| 182P04000S | Conn., FPC, 0.5mm, 40 cont, RA, Downside contact, SMT |
| 161C12121S | Res., 12.1K, 1%, 0402, 1/16W |
| 182P02000S | Conn., FPC, 0.5mm, 20 cont, RA, Downside contact, SMT |
| 182K10000S | Conn., B2B, 2x50 cont, P:0.8mm, Receptacle, H:3.7mm, SMT |
| 163K04330S | Res., Array, 33R, 8P4R, 5%, 0805 |
| 163K04102S | Res., Array, 1K, 8P4R, 5%, 0805 |
| 163K04103S | Res., Array, 10K, 8P4R, 5%, 0805 |
| 132D42200S | Diode, TVS, Dual USB 2.0 ESD protection, Low cap (1pF) |
| 151B10780S | Cap., 100uF, 6.3V, X5R, 20%, 1210, H=2.5mm |
| 111S40600S | IC, Memory, EEPROM, 93C46, 64x16, 2.7V-5.5V, DFN2x3mm |
| 161C10011S | Res., 1K, 1%, 0402, 1/16W |
| 116A32430I | IC, Interface, MAX3243,Industrial, RS232 drvr,5T3R, 3.3V,SSOP28 |
| 139A10130S | LED, Chip 3x1mm, R/A, Green, 16mcd, 20mA, 2.1V fwd |
| 139A10140S | LED, Chip 3x1mm, R/A, Orange, 3.2mcd, 20mA, 2.2V fwd |
| 383D52530 | Spacer, Hexagonal, M2x.4 thread, 4.14mm Length, Brass with tin coating |
| 182K00802 | Conn., RJ45, W/ 100B-T transformer & term res & 2KV cap |
| 116A34910S | IC, Interface. MAX3491, RS485/RS422, 10Mbps, 3.3V |
| 106C20000S | IC, Logic, MicroGate, NC7SZ00P5, 2-input NAND gate, SC70 |
| 190I00600S | Socket, Sim, SMT, 6pin |
| 161C14031S | Res., 140K, 1%, 0402, 1/16W |
| 151D22000S | Cap., 22pF, 50V, NPO, 5%, 0402 |
| 161C34801S | Res., 348R, 1%, 0402, 1/16W |
| 163K04750S | Res., Array, 75R, 8P4R, 5%, 0805 |
| 161D15025S | Res., 15K, 5%, 0603 |
| 163K04000S | Res., Array, 0R, 8P4R, 5%, 0805, Convex |
| 106C20320S | IC, Logic, MicroGate, NC7SZ32, 2 Input OR gate, SC70 |
| 170B00001S | SWITCH, Push-B,Tact, 6x6 mm, 4.3mm High, SMT |
| 131D03350S | Transistor, N-MOSFET, NDS335N,Vgs=100mOhm@2.5V,1.7A,20V |
| 210C30002S | Battery. Lithium, Cell 16x2.3mm with 2 SMT pads,3V,45mAh |
| 178A00200 | Terminal Block, Socket, 2P, 3.5mm pitch, Screw, RA |
| 182E10010S | Conn., CardBus, Top mount, 4.3mm standoff, no eject |
| 182E10011S | Conn., CardBus top part for dual conn., Top mount,4.3mm stanoff, no eject |
| 380J52080 | Screw, M2, Pan head, Philips,10mm length, Steel, Nickel or Zinc plating |
| 380J52110 | Screw, M2, Pan head, Philips,16mm length, Steel, Nickel or Zinc plating |
| 161C47511S | Res., 4.75K, 1%, 0402, 1/16W |
| 151D10000S | Cap., 10pF, 0402, NPO, 50V, 5% or 1% |
| 151D10100S | Cap., 100pF, 0402, NPO, 50V, 5% or 1% |
| 151D10200S | Cap., 1nF 0402, X7R, 50V, 10% |
| 161C15011S | Res., 1.5K, 1%, 0402 |
| 161C56211S | Res., 5.62K, 1%, 0402 |
| 161C10R05S | Res., 10R, 5%, 0402 |
| 102E80660S | IC, Logic, Bus switch, 2-bit, NC7WB66 |
| 180K01501 | Conn., DB15 HD, 3.08mm, RA, Fem, Blue, PC99, TH |
| 131A39040S | Transistor, NPN, MMBT3904 |
| 381A52000 | Nut, Hex, M2, Steel, Nickel or Zinc plating, 1.6-2.0mm width |
| 139A10010S | LED,Red,SMT,1206 |
| 182P05100S | Conn., FPC, 0.3mm, 51 cont, R/A, SMD |
| 185B10010 | Conn., Jumper Plug (shunt), 2.54mm (.1") spacing, Gold plated, Black color |
| 131D76040S | Transistor, P-MOSFET, IRF7604,Vgs(th)=-0.7V, 3A,20V |
| 374A12X20W | Label, 0.8"x0.5" (12.7x20.32mm), white, Polyester |
| 141B25600I | XTAL., Industrial, 25.000MHZ, 50PPM, 7x5mm SMT, -40°C+85°C |
| 116B81390S | IC, Periph., RTL8139DL, Fast Ethernet Controller, PCI, 3.3V |
| 180K00901S | Conn., Mini-USB, Type AB, SMT, 5 pin |
| 190C00200 | Conn., Header, 1x 2, 100MIL, TH, JP2, 3mm tail, 6mm mating length |
| 190C00300 | Conn., Header, 1x 3, 100MIL, TH, JP3, 3mm tail, 6mm mating length |
| 190C00600 | Conn., Header, 1x 6, 100MIL, TH, JP6, 3mm tail, 6mm mating length |
| 188C01153 | PCB, SB-X270 Rev 1.3 |
| 163I08822S | Res., Netwrk, 8.2K 10P8R, Bussed, 1206 pkg |
| 161D12001S | Res., 120R, 1%, 0603 |
| 147D60100S | Ferrite Bead, 0603, Z=330 Ohm@100Mhz,200mA, 0.5 Ohm DCR |
| 190C01000 | Conn., Header, 2x 5, 100MIL, TH, DR10, 3mm tail, 6mm mating length |
| 147D25170S | Ferrite Bead, 2518121217Y1,1812 SMD, 1.5A, (z)120ohm, (dcr) 0.05ohm |
| 190C01000S | Conn., Header, 2x 5, 100MIL, SMT, DR10, 6mm mating length |
| 151B10511S | Cap., 1uF, 6.3V, X5R, 10%, 0603 |
| 106O02300I | IC, PLL, CY2309,Ind., 9 outputs, 25-75Mhz, 3.3V |
| 190C04000S | Conn., Header, 2x20, 100MIL, SMT, 6mm mating length |
| 180K00900S | Conn. SD/MM card reader, SMT, Push-Push |
| 123V6R500S | IC, Volt reg., Step down, controller, Vimax=6.5V, TPS64202 |
| 123V6R000I | IC, Volt. Reg., Wide input 10-55V, Sync buck cont |
| 131D74150I | Transistor, P-MOSFET, 5.7A, 60V, RDS=65mOhm@Vgs=10V |
| 131D78500I | Transistor, N-MOSFET, 10.3A, 60V, RDS=22mOhm@Vgs=10V |
| 151E47560S | Cap., 4.7uF, 100V, X7R, 20%, 2220 |
| 151E10480S | Cap., 100nF, 100V, X7R, 20%, 1210 |
| 147A15001S | Inductor, 15uH, 4.5A, 27mOhm, SMT |
| 182P02400S | Conn., FPC, 0.5mm, 24 cont, RA, Downside contact, SMT |
| 190C01800 | Conn., Header, 1x18, 100MIL, TH, 3mm tail, 6mm mating length |
| 190C02602 | Conn., Header, 1x26, 100MIL, TH, 3mm tail, 6mm mating length |
| 182N10400 | Conn., PC/104,2x32+2x20, Male/Fem, Stacktrough, Pressfit |
| 374R16P16W | Label, 16mm Round, Paper, White |
| 151J10250S | Cap., 1nF, 2KV,SMD 1812, X7R, 10%, 7"REEL |
| 182K14000S | Conn., B2B, 2x70 cont., P:0.6mm, Rec, H:4mm |
| 132D56000S | Diode, Transzorb, 56V, Bi-Di, 600W |
| 151K10410S | Cap., 100nF 0603, X7R, 16V, 20% |
| 161C22121S | Res., 22.1K, 1%, 0402, 1/16W |
| 163E04151S | Res., Array, 150R 8P4R, 1206 (3.2x1.6mm) pkg |
| 190D12001 | Conn., PC/104 Plus, 120 pin, 2mm pitch, Stacktrough, Pressfit, 12.53 term len |
| 151K47210S | Cap., 4.7nF, 50V, X7R, 10%, 0603 |
| 107A83270S | IC, PLD, LC4032-75 |
| 161C20031S | Res., 200K, 1%, 0402, 1/16W |
| 124S82200S | IC, Analog switch, RYC8220-1M, Dual USB current limited switch, so-8 |
| 130E83500S | Diode, Schottky, MBRD-835L, 8A, 35V, DPAK |
| 161C71521S | Res., 71.5K, 1%, 0402, 1/16W |
| 147A2R201S | Inductor, 2.2uH, Isat=14A, 18Mohm DCR |
| 151C22310S | Cap., 22nF 0603, X7R, 25V, 20% |
| 161D10055S | Res., 10M, 5%, 0603 |
| 161HR0121S | Res, 0.012R, 1%, 2010, 1/2W (current sense) |
| 131A11970S | Transistor, PNP, 2SB1197K, Gen purpose, 0.8A |
| 203G00400 | Conn., Acc., PC/104 guide pins for 40P conn |
| 123V90000S | IC, Volt. Reg. Si9183DT-AD-T1, Adjustable, 150mA |
| 203G00640 | Conn., Acc., PC/104 guide pins for 64P conn |
| 161D17421S | Res.,17.4K, 1%, 0603 |
| 130E05402S | Diode, Schottky, sgl, BAT54WS, SOD-323 |
| 151B10620S | Cap., 10uF, 6.3V, X5R/X7R, 20%, 0805 |