# Image Updater

Image Updater	1

Overview	1

Nand Flash layout	1

System Installation Mode	1

System Update Mode	2

Batch Mode	2

## Overview

The Image Updater is an application targeted to run on a customized version of WinCE 6.0 that is supplied with the package (wince_updater.nb0) and performs the following actions:

- Installation of new NK.bin on the device

- Update of NK.bin only

- Batch mode operation

## Nand Flash layout

To understand what is performed by the Updater knowledge of the Nand Flash layout is needed. The following table specifies what each block on the Nand Flash represents:

| Block Number | Designated function |
| --- | --- |
| 0-5 | Uboot area |
| 6-14 | Splash Screen reserved area |
| 15 | Control Block1 |
| 16 | Control Block 2 |
| 17 | MBR and start of OS |

Starting from Block 17, the Nand Flash is divided into two logical partitions. First, a BINFS parturition that is large enough to hold two NK.bin files, each of up to 35 Mb (A hard coded value in the updater). Second, a FATFS partition that is the data partition that is mounted when the image loads.

NOTE: The last 127 blocks of the Nand are reserved for Bad Blocks management.

## System Installation Mode

The Installation procedure includes the following steps:

- Low Level Format of NAND Flash

- Prepare BINFS partition for NK.bin

The BINFS partition is allocated for maximum NK.bin of  ~35 MB.

The CM-X300 also allocates space for another NK.bin file of maximum size in order to implement a FailSafe update mechanism during which any failure does not defect the module completely, instead it will load with the old image again.

- Prepare FAT partition for data storage.

The regular installation process requires the following user interaction:

- Choose the location of NK.bin file with the "Browse" button.

- Start the installation process with the "Start" button.

- Press the “OK” button to reboot.

## System Update Mode

The System Update mode allows updating NK.bin only, without loosing the information on the data partition. The UI interaction is almost the same as in the System Installation mode. The only difference is the option to override FailSafe update, in which case the system performs a full installation with the quick format option.

The following is a screen shot of the above situation:

NOTE:

After a NK.bin update the registry changes (from a previous image) will be lost.

## Batch Mode

This mode enables an automated deployment of the image to the device.

When the Image Updater starts, it looks for a script file named “script.xml” and executes it. It first looks for the file on the USB Flash drive (seen as \Hard Disk\) and, if there it is not available, it looks for the file on the SD card (seen as \Storage Card\). If it fails to find the file on neither media, it initializes the GUI, which was previously described.

During the script execution the update progress is displayed on the LCD.

Note: You can run the batch mode installation without connecting the LCD.

The script file is in .xml format and exports the following options:

- FORMAT_NAND – formats the NAND Flash and prepares BINFS partition

<OPERATION category="FORMAT_NAND">

<OrderToPerform>1</OrderToPerform> <QuickFormat>TRUE</QuickFormat>

</OPERATION>

| Field Name | Field Description |
| --- | --- |
| OrderToPerform | Order of the operation execution in script |
| QuickFormat | Whether to perform quick format of NAND |

- UPDATE_OS – updates NK.bin file in BINFS partition

<OPERATION category="UPDATE_OS">

<OrderToPerform>2</OrderToPerform>

<FileName>\Hard Disk\NK.BIN</FileName>

</OPERATION>

| Field Name | Field Description |
| --- | --- |
| OrderToPerform | Order of the operation execution in script |
| FileName | Absolute path to NK.bin file |

- Eboot_cfg – Sets the Eboot configuration

<OPERATION category="Eboot_cfg">

<OrderToPerform>8</OrderToPerform>

<!--Cpu Frequency is one of the following:-->

<!--3: 208 MHz-->

<!--4: 416 MHz-->

<!--5: 624 MHz-->

<!--0: IPM managed-->

<CpuFreq>4</CpuFreq>

<EnableDhcp>True</EnableDhcp>

<!--The IP address field is only relevant in case the DHCP is disabled-->

<IPAddress>10.1.1.70</IPAddress>

<!--Usb device is only relevant for PXA300-->

<!--The following are the possible values:-->

<!--0: USB 1.1-->

<!--1: USB 2.0-->

<UsbDevice>2</UsbDevice>

<!--This sets the number of seconds Eboot waits before starting to Launch-->

<BootDelay>3</BootDelay>

<EnableFFUART>False</EnableFFUART>

<EnableBTUART>False</EnableBTUART>

<EnableBattery>False</EnableBattery>

<EnableSPI>False</EnableSPI>

<EnableBacklight>True</EnableBacklight>

<!--The codec driver controls Touch and Sound functionality-->

<EnableAC97Codec>True</EnableAC97Codec>

<!--The following decides whether STUART is used for debug or as a UART-->

<!--False means it will function as debug serial-->

<EnableSTUART>False</EnableSTUART>

<!--The following options are possible for WiFi/BT activation:-->

<!--1: Both Disabled-->

<!--4: Both enabled-->

<EnableWiFiBT>1</EnableWiFiBT>

</OPERATION>

- REBOOT – performs reboot of the device

<OPERATION category="REBOOT">

<OrderToPerform>3</OrderToPerform>

</OPERATION>

| Field Name | Field Description |
| --- | --- |
| OrderToPerform | Order of the operation execution in script |

The sequence of the proposed operations can be treated as the NK.bin installation procedure described in System Installation chapter.

See the "script_example.xml" file that is located in the \ImageFiles folder for additional details.

Running the Script Mode with just the update operation and without formatting is equivalent to the GUI option of FailSafe update (leaving the check box unchecked). This is optional only after one full installation was done.