CompuLab SysConf utility Documentation

- Purpose

Sysconf is a powerful command line tool. The Purpose of the SysConf utility is primarily as an extensive debug tool. This tool is intended for advanced developers.

- Interface & Examples

The following is the output when running SysConf without arguments:

Usage:  SysConf         - to print this help

SysConf  <name>         - to dump named register

SysConf  <address>      - to dump memory address (prefix with 0x for hex 			     number)

SysConf  <name> <value> - to set named register

SysConf  ALL    - to dump all registers

SysConf  LCD    - to dump all LCCR registers

SysConf  AC97 <address> <value> - to update AC97 Codec register

SysConf  AC97_ALL       - to dump all AC97 Codec registers

SysConf  DA9030 <address> <value>  - to update DA9030 register

SysConf  DA9030_ALL -to dump all DA9030 Codec registers

SysConf  GPIO <pin number>      - to print all

the relevant gpio data for the specified pin

SysConf  GPIO <pin number> L<#> D<#> RE<#> FE<#> AF<#>

- to manually set a certain pin with desired config

args meaning: L-Level(set=1/clear=0) D-direction(1 for output)

RE-rise edge set AF- between 0-3

NOTE: you can change varied number of params

Example: SysConf GPIO 86 L1 D0 AF3

SysConf  GPIO - to dump all GPIO registers

SysConf  DEVICE <name> START - to activate a device

SysConf  DEVICE <name> STOP - to activate a device

SysConf  DEVICE <name> RESTART - to activate a device

<name> - is the alias as specified in the documentation

SysConf  ALLDEVICES - to print all active devices

SysConf  ALLALIAS - to print all active devices with Alias name

SysConf  NETIF  - to list all available network interfaces

SysConf  NETIF <name> <cmd>  - to START/STOP/RESTART a network 			     interface

for example - SysConf NETIF WLAN1 STOP

Now we will go through each (non self explanatory) option with elaborated explanation and an example:

- This tool is NOT case sensitive.

- The link for the PXA-27x developer manual (you need to register to Marvell's extranet to be able to download it), Link. (In case the link is broken search for “PXA-27x manual in Marvell's extranet)

  - SysConf <name> where name is a name of a register as specified in the PXA-270 documentation, for example: SysConf GRER0

  - SysConf <address> where address is the address of a register as specified in the PXA-270 documentation, for example: SysConf 0x40e00000, this will print GPLR0.

  - SysConf GPIO <pin number> is used when you want a friendly printout of a specific GPIO pin, for example: SysConf GPIO 86, will give the following output:

Level=HIGH

Direction=INPUT

Rising edge=DISABLED

Falling edge=DISABLED

Edge transition NOT DETECTED

AlternateFunction=0

  - SysConf  GPIO <pin number> L<#> D<#> RE<#> FE<#> AF<#> is used to set the different attributes of a GPIO pin, where L is the level of the pin (high or low), D is the direction (input=0 and output=1), RE is whether to set trigger for rising edge, FE is whether  to set trigger for falling edge, AF is to determining the alternate function of the pin (values are from 0-3). For example: 	         SysConf GPIO 86 L1 D1 AF0– this will set pin 86 to be output with 3.3v and alternate function 0.

SysConf GPIO 86 D0 RE1 AF1 – will set the pin to be input and to have a trigger of rising edge, also we set the alternate function to 1.

NOTICE: As can be seen in the examples the number of attributes you set is variable, e.g. It is not necessary to set all the options.

  - SysConf  ALLALIAS – prints all active devices which have an alias name. For example the output can be:

1: WLN1: -> Drivers\BuiltIn\SPI, Alias=WLAN, key name is: 50

2: UFN1: -> Drivers\BuiltIn\Usbfn, Alias=USBFN, key name is: 39

3: COM3: -> Drivers\BuiltIn\Serial4, Alias=SERIALD, key name is: 35

4: COM2: -> Drivers\BuiltIn\Serial3, Alias=SERIALC, key name is: 30

5: COM1: -> Drivers\BuiltIn\Serial2, Alias=SERIALB, key name is: 29

6: SDH1: -> Drivers\BuiltIn\SDHC, Alias=SDCARD, key name is: 27

7: HCD2: -> Drivers\BuiltIn\PCI\Instance\OHCI2, Alias=OHCIITE, key name is: 26

8: NDS1: -> Drivers\BuiltIn\PCI\Instance\RTL81392, Alias=RTL8319, key name is: 25

9: HCD1: -> Drivers\BuiltIn\OHCI, Alias=OHCIPXA, key name is: 21

10: DSK1: -> Drivers\BuiltIn\NorFlashV3, Alias=NOR, key name is: 18

11: I2C1: -> Drivers\BuiltIn\I2C, Alias=I2C, key name is: 14

12: GIO1: -> Drivers\BuiltIn\GIO, Alias=GPIO, key name is: 11

13: DSK0: -> Drivers\BuiltIn\NandFlashV3, Alias=NAND, key name is: 02

  - SysConf  DEVICE <name> STOP – where name is the alias name as can be fetched by using the command SysConf ALLALIAS. This command stops a device driver and unloads it from the system. For example: SysConf DEVICE WLAN STOP

  - SysConf  DEVICE <name> START – where name is the alias name as can be fetched by using the command SysConf ALLALIAS. This command starts a device driver and loads it to the system. For example: SysConf DEVICE WLAN START

  - SysConf  ALLDEVICES – prints all active devices which have a prefix name. An output example:

1: TEL1: -> services\TELNETD\NewSession, key name is: 62

2: NTP0: -> Services\TIMESVC, key name is: 59

3: SMB0: -> Services\SMBServer, key name is: 58

4: TEL0: -> Services\TELNETD, key name is: 57

5: FTP0: -> Services\FTPD, key name is: 56

6: HTP0: -> Services\HTTPD, key name is: 54

7: UIP1: -> uiproxy, key name is: 52

  - SysConf  NETIF – prints a list of all the network interfaces as can also be seen from the GUI: Start->Settings->Network and Dial-Up Connections.

  - SysConf  NETIF <name> <cmd> where name is taken from the list printed by the previous command, and cmd can be STOP, START or RESTART, For example: SysConf NETIF WLAN1 STOP , after this command is issued the interface should be disabled in the GUI