Win CE 6.0 for Exeda

Getting Started Guide

This version of Windows CE is intended for evaluation purposes only. It may be used for up to 30 days. After this period, for continued use, the user must buy an appropriate Windows CE license.

Release Notes	3

Introduction	4

WinCE 6.0 Demo Distribution Highlights	4

Exeda Features Supported	4

Demo Distribution Contents	4

WinCE 6.0 Demo Distribution Installation	6

Installation Steps	6

Installation Troubleshooting	7

Installation Customization	7

Get Connected with ActiveSync	8

Demo Distribution Overview	9

Demo Distribution Versioning Support	9

RAM	9

NAND Flash Disk	9

Root Partition	9

Persistent Registry	9

Fail-Safe Data Storage	9

Wireless Interface Control	10

USB	10

Touch Screen	10

Touchpad	10

Keyboard	10

LCD Resolution	11

Backlight	11

Suspend / Resume	11

Hardware Shutdown	11

Battery and Charging	11

Serial Ports	12

Telit GPRS / GSM Cellular Modem Evaluation	12

GSM Evaluation	12

GPRS Evaluation	13

General Cellular Module Evaluation	14

GPS	14

Bluetooth	15

Camera Evaluation	15

Splash screen Customization	16

Reboot	16

Application Development with Demo Distribution	17

Running a User Program from NAND Flash on Startup	17

Customizing Demo Distribution	17

Driver Customization	18

Touch Screen Driver	18

Battery Driver	18

Network Drivers	19

Customizing System Power States	19

Helpful Utilities	19

RegEdit Utility	19

SetPowerState Utility	20

AutoLaunch Utility	20

SysConf Utility	20

Notes and Errata	21

# Release Notes

| 08-Jun-2009 BUILD: 1.01.1050 | - First Release of Win CE 6.0 for Exeda |
| --- | --- |

# Introduction

This document gives a brief introduction to the Windows CE 6.0 Demo Distribution for Exeda. It describes the installation process and Exeda-specific customizations.

This document is intended for Windows CE/Mobile application developers and assumes familiarity with the Windows CE/Mobile operating system.

# WinCE 6.0 Demo Distribution Highlights

The WinCE 6.0 Demo Distribution is based on a standard WinCE 6.0 R2 Internet Appliance project with the addition of Control Panel applets for Exeda feature control and Freeware applications.

## Exeda Features Supported

The following Exeda devices are supported in the Demo Distribution:

- NAND Flash Disk (Microsoft FAL/FMD Flash Driver model)

- TFT LCD (pre configured for Toppoly TD035STEE1 TFT 480x640)

- Audio

- Touch Screen

- SD/SDHC/MMC CARD

- Wi-Fi 801.11b/g

- Bluetooth

- Battery and Charger

- Suspend / Resume

- Backlight

- Keypad

- Touchpad

- Ethernet (via EB-X270 extender board)

- VGA (via EB-X270 extender board)

- USB OTG (both USB Slave/Host functionality for primary USB connector)

- GPRS/GSM modem accessible via COM1

- GPS accessible via COM2

- Serial Port (via Exeda serial port connector) accessible via COM3

- RIL Driver used by Demo Dialer application

- Quick capture interface

## Demo Distribution Contents

The following WinCE 6.0 R2 standard components are included in the Demo distribution:

- File system Explorer

- ActiveSync

- AYGShell (allows the user to run some Windows Mobile applications)

- Internet Explorer

- Windows Messenger

- Media Player (recommended for sound playback)

- TCPMP Player (recommended for divx/mpg4 video playback)

- ItaskMgr  (a PC-like task manager)

- WordPad

- Games

- Telnet Server

- OBEX

- All MS Bluetooth Profiles, not including the DUN Profile

- Location Framework

- GPSView (GPS configuration utility)

The following custom Exeda components are included in the Demo Distribution:

- Backlight Control Panel Applet

- Display Rotation Control Panel Applet

- Wireless Communication Control Panel Applet

- Keypad Configure Control Panel Applet

- A2dpMgr (Bluetooth A2DP sample)

- DemoDialer (GSM cellular demo dialer)

- CLTerm (serial port evaluation)

- CameraApp application (demonstrate camera capabilities of Exeda)

The detailed list of standard WinCE6.0 R2 components included in the Demo Distribution is specified in “\Docs\Included Components.html”

# WinCE 6.0 Demo Distribution Installation

This chapter describes the WinCE 6.0 Demo Distribution installation using LiveDisk.

Download the latest Compulab WinCE 6.0 package from:

http://www.compulab.co.il/exeda/html/exeda-developer.py

## Installation Steps

In order to install the WinCE 6.0 Demo Distribution, perform the following steps:

- Copy the contents of the LiveDisk directory from the package to a USB flash drive.

- Insert the USB flash drive into the “USB-A to mini-USB” adapter supplied with the Exeda Eval Kit, and connect the other side of the adapter to the Exeda USB OTG port (primary USB).

NOTE: Ensure that there is no SD/MMC card inserted in the device together with the USB flash drive

- Power up the Exeda while holding down the ON/OFF button. You may release the button 2-3 seconds after you see the Exeda logo on the LCD screen.

NOTE: A good indication of successful boot progress is the blinking of the USB flash drive activity LED (if present)

- Wait until the system boots into the Image Updater OS (approximately 1 minute). The system boot is finished when you see the desktop screen below.

- By default, the installation process will start automatically in unattended mode.

Installation is performed by the Image Updater application executing the installation script file (script.xml) located on the USB flash drive

- Upon successful WinCE 6.0 Demo Distribution installation (approximately 3 minutes), the system will automatically restart and boot the WinCE 6.0 Demo Distribution

## Installation Troubleshooting

- Ensure that no MMC/SD card is inserted during LiveDisk installation via the USB flash drive

- If Exeda fails to recognize your USB flash drive, try the following suggestions:

  - Reset Exeda

  - Format the USB flash drive with FAT16

  - Try another USB flash drive

- If your installation fails, please send us the installation log with your support request.

In order to obtain a installation log, perform the following steps:

  - Connect the "DB9-F to Ultra Mini Plug" supplied with the Eval Kit to the COM port of your host PC.

NOTE: If you don’t have a COM port on your host PC, use the USB to Serial third party solution

  - On your host PC, open a terminal application (such as Hyper Terminal or IVT Terminal). Use the following parameters to setup your connection with Exeda:

    - COM port: number of COM port that is attached to Exeda

    - baud rate: 115200

    - data bits:   8

    - stop bit:	     1

    - parity:	      none

    - disable any flow control options

(Important if you want to type something in the terminal console)

  - If the serial link is established properly, you will see the following output in your terminal window upon device power on:

U-Boot 1.2.0-eXeda (Apr  1 2009 - 15:00:25)

  - Send us the contents of your terminal window during the installation process

## Installation Customization

The \custom folder in your LiveDisk directory contains predefined .xml files for different types of installations. The following installation types are available:

- Development Installation (SCRIPT_DEVELOPMENT.XML)

This type of installations allows debug messages to a serial console (to ease debugging and installation troubleshooting) resulting in disabling the serial port driver (COM3) in WinCE

- Production Installation (SCRIPT_PRODUCTION.XML)

This type of installation is intended for shipping or when your design requires the use of a serial port driver (COM3) in WinCE

NOTE: The default script.xml executes Development Installation

In order to customize the installation process, overwrite the script.xml file in the LiveDisk directory with one of the templates from the \custom directory.

# Get Connected with ActiveSync

ActiveSync is the program that allows you to synchronize data and information between your desktop computer and your Exeda. To synchronize using ActiveSync, you must install ActiveSync on your PC before you connect the Exeda to the PC.

Download ActiveSync from Microsoft's website: (http://www.microsoft.com/windowsmobile/activesync/activesync45.mspx

Read the ActiveSync Troubleshoot Guide about network security configurations for your workstation to allow ActiveSync usage:

(http://www.microsoft.com/windowsmobile/help/activesync/default.mspx)

The Demo Distribution currently supports ActiveSync via a USB port.

In order to setup the connection, perform the following steps:

On Host (your PC)

- Ensure that ActiveSync is installed

- Ensure that ActiveSync synchronization via USB is enabled in the Connection Settings dialog box

On Target (Exeda)

- Connect “USB A plug to mini-USB B plug” (ActiveSync) cable to the Exeda.

- The ActiveSync connection will start automatically and you will see the connection status on your Exeda desktop.

# Demo Distribution Overview

The Demo Distribution image is packed into binary image file nk.bin and is stored in the NAND Flash during LiveDisk installation.

## Demo Distribution Versioning Support

The Demo Distribution versioning information is accessible by clicking Start >> Settings >> Control Panel >>System>>Copyrights. It includes the SVN revision number, commit date and actual Demo Distribution build date.

The Demo Distribution Versioning information must be included in each support request.

## RAM

The Exeda is equipped with 128 MB of RAM.

Approximately 80 MB is available for applications’ usage while the Demo Distribution is running.

## NAND Flash Disk

The Exeda is equipped with one or two NAND flash chips that can contain up to 16 GB of storage. Practically, a slightly lower amount of NAND Flash is available for the Demo Distribution’s storage, due to NAND Flash bad blocks.

If the two NAND Flash chips are present, they are treated transparently as a single logical disk to the user.

During the Demo distribution installation, the NAND Flash is automatically partitioned and formatted according to the following layout:

- BINFS partition to store nk.bin (plus 20% to allow future growth), approximately 42 Mb

- TFAT data partition for fail-safe data storage.

## Root Partition

The Demo distribution is built with a “ROM-only File System” option.

This means that the NAND Flash data partition is mounted as the root partition and all programs that you install will be stored in the NAND Flash.

## Persistent Registry

The Demo Distribution is built with a “Hive-Based Registry” option.

This means that the system registry is stored in the NAND Flash data partition, allowing the registry configuration to be saved during power-off.

Demo Distribution is configured with a “Lazy Flush” registry policy - the registry settings are stored in RAM and are flushed to the NAND Flash every 30 seconds. If you make any changes to the registry, allow 30 seconds before either turning the power off or performing a hardware reset in order to save the settings on NAND Flash.

## Fail-Safe Data Storage

The Demo Distribution uses the Microsoft FAL/FMD driver model to access the NAND Flash.

This allows using the TFAT (Transaction Safe FAT) as a root partition file system which guarantees data consistency during asynchronous power off events.

#### Networking

The Demo Distribution is configured to use DHCP to get network settings for Ethernet (Davicom DM9000) and Wi-Fi (Marvell GSPI8686) adapters. The network settings can be changed in the Control Panel by clicking Start >> Settings >> Network Connections.

## Wireless Interface Control

The Demo Distribution allows control of the wireless devices power state with the help of the Wireless Config control panel applet, accessible by clicking Start >> Settings >> Control Panel >> Wireless Config.

The following wireless devices are currently supported:

- Wi-Fi

- GPS

NOTE:

- The Telit GSM/GPRS module’s power state is controlled by the RIL driver and is currently always on.

- The Bluetooth power state mirrors the Wi-Fi power state.

This behavior is closely related to hardware design and will remain unchanged in the future.

## USB

The Demo Distribution currently supports USB 1.1 OTG functionality on Exeda’s primary USB connector.

Other USB ports are configured as host only.

## Touch Screen

The Demo Distribution is calibrated for the Toppoly TD035STEE1 LCD touch screen.

To fine-tune the calibration, run the touch screen calibrator on the first boot by tapping

Start >> Settings >> Control Panel >> Stylus >> Calibration.

Read more about touch screen calibrator registry settings at:

http://msdn2.microsoft.com/en-us/library/ms926952.aspx

## Touchpad

The Exeda supports a touchpad that is recognized as a standard mouse by Windows CE.

Any speed and accuracy adjustments can be made with standard mouse API’s.

## Keyboard

Exeda supports a 62-key QWERTY keyboard.

Presented below are the custom key combinations specific to the Exeda keyboard:

- Left Shift + Right Shift results in CAPS Lock

- SHIFT + Backspace results in DEL

- ALT + combinations are listed in the following table:

| ALT + | Result |
| --- | --- |
| 1 | ` |
| 2 | ~ |
| 8 | _ |
| 9 | [ |
| 0 | ] |
| - | = |
| U | ¦ |
| I | \ |
| O | { |
| P | } |
| L | " |
| ; | ' |
| . (period key) | < |
| / | > |
| SPACE | INSERT |
| LEFT | HOME |
| RIGHT | END |
| DOWN | PAGE DOWN |
| UP | PAGE UP |

## LCD Resolution

The Demo Distribution is configured for Toppoly TD035STEE1 TFT portrait LCD.

The default resolution is 480x640 with 16-bit color depth and landscape orientation (270 degrees rotated).

## Backlight

The Exeda supports 7 levels of backlight brightness.

You can modify the backlight level by tapping Start >>  Settings >> Control Panel >> Backlight Level.

## Suspend / Resume

Suspend is the most power-efficient state supported by Exeda.

The system goes to suspend almost instantaneously. The whole system’s state is saved to the SDRAM, the peripherals are either put into idle mode or disabled. On exit, the peripherals will undergo initialization and state restoration.

You can enter suspend by pressing the ON/OFF button on the Exeda. Pressing it again will start the resume process.

Resume takes approximately three seconds to complete for the Demo Distribution depending on which external devices are connected and how long it takes to initialize each one.

During Suspend all external devices are powered off (except for the GSM/GPRS modem).

## Hardware Shutdown

By pressing the ON/OFF button for more than 2 seconds, you will see a popup dialog that asking for confirmation in order to proceed with device hardware shutdown.

To turn on a device that was previously put in hardware shut down, press the ON/OFF button.

NOTE: Pressing the ON/OFF button for more than 5 seconds will shutdown the device automatically (can be used when a device is not responding).

## Battery and Charging

The Exeda is equipped with a 3000mAh Li battery.

You can monitor battery power by tapping Start >>  Settings >> Control Panel >> Power.

There are two charging options supported on Exeda:

- Charge using the supplied “DC plug to mini-USB" adapter and DC supply.

This is the fastest charging option with a charge current of 1000mA (3 hours to fully charge the battery).

- Charge using the PC sync cable (ActiveSync cable).

This option is limited to a 500mA charge current (6 hours to fully charge the battery) due to the USB PC port’s current limitation.

During charging, you will see the “charging in process” icon on the taskbar.

For more details, refer to section 5.2 of the “Exeda Manual”

To maximize battery life:

- Turn off the backlight when the device is not in use.

You can set the backlight policy by tapping

Start >>  Settings >> Control Panel >>  Display >> Backlight

- Suspend frequently.

Press the sleep/resume (SW2) button to enter Suspend Mode when Exeda is not in use, even for a short time.

- Turn off wireless interfaces when not in use.

You can control the power consumption of wireless devices by tapping

Start >>  Settings >> Control Panel >>  Wireless Config.

## Serial Ports

The Demo Distribution supports three serial ports, two of which are dedicated to on-board functions:

- GPRS Modem on COM1  (at 38400 baud)

- GPS Unit on COM2 (at 9600 baud)

- Fully Functional fast UART on COM3, accessible via RS232 serial connector

Ensure that you have performed the "Production Installation" before evaluating the COM3 (see the Installation Customization" section above).

You can evaluate COM3 behavior with the help of the CLTerm utility located in "Start >> Programs >> Tools >> CLTerm" (see "General Cellular Module Evaluation" section for more information regarding CLTerm usage).

NOTES:

- The COM1 Serial driver does not support multiplexing. Thus, if you want to use a GPRS modem in your application, disable the Bluetooth DUN profile (disabled by default).

## Telit GPRS / GSM Cellular Modem Evaluation

Exeda is equipped with a Telit GC864-QUAD cellular modem.

The current demo software allows evaluating the following modem capabilities:

- GSM dialer (put and receive GSM calls)

- GPRS connection

- Advanced AT commands evaluation

## GSM Evaluation

Ensure that you have connected the GPRS antenna and have inserted a SIM card before the evaluation.

In order to evaluate the GSM cellular capabilities of Exeda, use the Demo Dialer application located in "Start >> Programs >> Demo Dialer". This self-explanatory application allows you to place and receive voice calls through a GSM cellular network.

NOTES:

- The dialer application is implemented on top of the RIL driver that is loaded automatically and turns on the cellular modem on startup.

- The phone number specified in the "Enter Phone Number" field is passed as is to the GSM modem without any prefix additions.

- If you are evaluating in US frequency band (GSM-850 GSM-1900), add the following registry key with the Remote Registry Editor:

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\RIL]

"ComInitString"="#BND=3"

- The source code of the application can be found in x270-sp-wince6-img\Demo Applications\Demo Dialer

## GPRS Evaluation

You can evaluate the GPRS data connection via Windows CE TAPI interface.

Use the "Start >> Settings >> Network and Dialup connection >> GPRS" shortcut in order to establish the Windows CE TAPI based connection.

NOTES:

- The predefined GPRS shortcut uses the COM9 serial port provided by the MUX driver

- The GPRS shortcut is pre-configured with Israeli Orange operator GPRS dial-up settings:

  - Dial to local number *99***1#

  - Add +CGDCONT=1,"IP","INTERNET" under "Extra Setting" of Call Options tab.

- For US frequency band (GSM-850 GSM-1900) add "#BND=3" under "Extra Settings" of the GPRS connection Call Options tab.

If you want to customize the GPRS connection settings, please read the following article describing in detail the Windows CE TAPI GPRS connection setup:

http://www.windowsfordevices.com/articles/AT8185724467.html

## General Cellular Module Evaluation

The Telit cellular modem can be accessed by sending a direct AT command to COM8 (emulated serial port by mux driver).  The CompuLab terminal application can be used to facilitate AT command evaluation. It is located in "Start >> Programs >> Tools >> CLTerm".

In the CL Terminal application, press the Config button to select the COM8 port and the baud rate 57600. After opening the COM8 port with the Open button, you can enter text in the Xmit field.

Refer to the Telit AT command set (http://www.telit.com/module/infopool/download.php?id=542) for advanced GPRS / GPS evaluation.

NOTES:

- No response will be printed upon COM8 command transmit. Response is automatically captured by the Telit RIL driver

- The Telit GPRS modem is configured to use European (GSM-900 GSM-1800) bands.

Use the "AT#BND=3" command to set it to the US band (GSM-850 GSM-1900).

For more information, please read the Telit AT command reference (see link above).

## GPS

Ensure that you have connected the GPS antenna before the evaluation.

The Demo Distribution includes a “GPS Viewer” application located at Start >> Programs >> GPS Viewer that can be used for Exeda GPS evaluation.

In the GPS Viewer, select COM2 and 9600 baud rate, and click “Open GPS” to open a connection to the device. You can see the satellite positions on the GPS status tab.

The Demo Distribution includes Location Framework that allows you to write GPS-aware applications. Read more about Location Framework at the following link:

http://msdn2.microsoft.com/en-us/library/aa914661.aspx

## Bluetooth

Before you begin, turn on Wi-Fi power via Start >> Settings >> Control Panel >> Wireless Config applet (remember that the Bluetooth power state mirrors the Wi-Fi power state on Exeda).

The Demo Distribution includes the following Microsoft Bluetooth profiles:

- Object Push Profile (OBEX)

- File Transfer Profile

- Human Interface Device Profile

- LAN Access Profile

- Personal Area Network Profile

- Hands-free and headset Profiles

You can find, pair and connect other devices using the Bluetooth Manager located at Start >> Settings >> Control Panel >> Bluetooth Device Properties.

In order to evaluate profile functionality, you should write a Bluetooth Profile-aware application. You can find many Microsoft Bluetooth samples in WinCE 6.0 shared source code that accompanies the Platform Builder installation (C:\WINCE600\PUBLIC\COMMON\OAK\DRIVERS\BLUETOOTH\SAMPLE).

In order to evaluate Bluetooth connectivity, the following devices were used:

- Exeda with Demo Distribution

- PC with BlueSoleil USB dongle

- Motorola HT820 Bluetooth Stereo Headphones

The following Bluetooth usage scenarios were tested with Demo Distribution on Exeda:

- PC can exchange OBEX object with Exeda

- PC can exchange files with Exeda via FTP Profile

- Exeda can stream audio to Bluetooth Headphones

For Bluetooth Audio connection, the AdpMgr Sample, located at Start >> Programs >> A2dpMgr

was used.

NOTE:

Do not enable the Bluetooth DUN profile in the Catalog as it locks the COM1 for Bluetooth DUN service only, thus disabling the COM1 usage for the GPRS/GSM modem.

## Camera Evaluation

You can evaluate the Exeda 2 mega pixel camera with the help of the CameraApp demo application, located at Start >> Programs >>CameraApp. This application allows you to preview a live video (a grey rectangle in the picture below) and to capture pictures (in 480x360 resolution) by pressing the "Take Snapshot" button. When capturing is complete, the captured image is saved in .jpg format under "\My Documents" folder (following the exeda_cam_date.jpg naming convention). Use the exit button to close the application.

## Splash screen Customization

You can customize the splash screen supplied with the Demo Distribution.

The 8-bit  ".bmp" image should be located in \LiveDisk\splashscreen.bmp during the installation process.

Notes:

- Only 8-bit .bmp images are supported by the splash screen.

- With the current U-Boot release, the splash screen image displays artifacts. This problem will be fixed in future releases.

## Reboot

Use the “Software Reboot” shortcut on the desktop to reboot.

Before reboot, the system is gracefully shut down including a flush of the registry.

## Application Development with Demo Distribution

This package includes MFC/.NET demo applications developed using VS2005, located in the “Demo Applications\” directory in the image archive. The applications are:

- RTCResume

This application demonstrates suspend/resume functionality using the RTC alarm event.

- DemoDialer

This application demonstrates the GSM voice ability of Exeda. It allows you to place and receive voice calls through a GSM cellular network

- CLTerm

This application allows evaluating the serial port capabilities of Exeda.

Primarily used for sending custom AT commands to the Telit module (COM8)

- CameraApp

This .NET application demonstrates the camera capture/preview capabilities.

Can be used as reference code for other .NET based applications using platform access (send IOCTL’s)

Before you begin application development using Microsoft Visual Studio 2005 (VS2005), you must install CompuLab SDK for Demo Distribution. The SDK installer “SP_X270_SDK.msi” is located in the SP_X270_SDK directory of your distribution archive.

See “\Docs\Developing WinCE 6.0 Application .doc” for the details of the application develop/debug/deploy process.

## Running a User Program from NAND Flash on Startup

You can start a custom application on system startup with the help of the AutoLaunch utility integrated in the Demo Distribution.

The AutoLaunch monitors the following registry key:

[HKEY_LOCAL_MACHINE\Startup]

“Process1” =”Application.exe param1 param2”

“Process1Delay”=dword:delay ;in milliseconds

See the “AutoLaunch Utility” section for details and an example.

# Customizing Demo Distribution

It is possible to customize the Demo Distribution behavior using the command line parameters from bootloader (U-Boot). You can define the command line in U-Boot by typing:

- setenv  cmdline “param1 param2 param3 ..”  (parameters are not case sensitive)

- saveenv

The following parameters are currently supported:

| HIVECLEAN | Clean registry hive on boot |
| --- | --- |
| FORMATUSER | Format user partition on boot |
| NO(driver alias) | Disables the driver by its alias name |
| ADD(driver alias) | Enables the driver by its alias name. The self-explanatory driver alias names are: SDCARD, SERIAL, TOUCH, GPIO, I2C, USBFN, WLAN, OHCIPXA, DM9000, AUDIO. |

NOTE: In order to remove the "cmdline" variable, type:

- setenv  cmdline “”

- saveenv

# Driver Customization

## Touch Screen Driver

The Exeda touch screen driver can be configured for different application’s needs with the following registry key:

[HKEY_LOCAL_MACHINE\HARDWARE\DEVICEMAP\TOUCH]

"TouchSensitivity"=dword:8

"PressureFilter"=dword:0

"PressureTreshold"=dword:c8

"PenMoveFilter"=dword:1

"PenMoveSensitivity"=dword:32

"SpikeFilter"=dword:0

"PenUpPollingInterval"=dword:28

The touch screen driver is implemented to detect the "pen down" interrupt and then poll for a "pen up" event. The polling interval can be specified in milliseconds in the "PenUpPollingInterval" value (defaults to 25 Hz).  Each sample should be filtered due to the noise sourced from the LCD, backlight and mechanical vibrations. Failing to do so can drastically reduce the accuracy and reliability of the entire touch screen system. The tradeoff of heavy filtering is touch sensitivity needed, for example: for drawing with the touch screen.

The driver implements the following filters:

- Spike Filter calculates the average of the measured samples and discards the sample, whose coordinates are most different from the sample's average. It can be set on/off by providing "1"/ "0" as the "SpikeFilter" value.

- Pen Movement Filter gets the coordinates of the touched point and compares them to coordinates of the previously touched point. If their difference is smaller than the adjusted threshold ("PenMoveSensitivity" value), the filter treats this point as noise and discards it.

The filter can be set on/off by providing "1"/ "0" as the "PenMoveFilter" value.

- Pressure Threshold Filter checks the sample's pressure value and accepts the sample only if this value is greater than the adjusted threshold ("PressureTreshold" value).

The filter can be set on/off by providing "1"/ "0" as the "PressureFilter" value.

NOTES:

- By default, the Spike and Pen Movement filters are enabled. If your application requires better response and accuracy from the touch screen, decrease the "PenMoveSensitivity" value.

- "TouchSensitivity" controls the codec hardware touch sensitivity. Set it to 1 for maximum sensitivity and 63 for minimum sensitivity

## Battery Driver

The battery charging process can be tuned to support different types of batteries, by the following registry key:

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\Battery]

"CVChargerTimeout"=dword:2

"CVChargerStartVoltageTreshold"=dword:ED8

"CCChargerStartVoltageTreshold"= dword:F6E

The battery charging process starts with "constant current" phase and ends with "constant voltage" phase. In order to start each of the above charging phases, the battery voltage (in milli volts) should be lower than "CCChargerStartVoltageTreshold" and "CVChargerStartVoltageTreshold", respectively.

With the "CVChargerTimeout" registry value, the total time for the "constant voltage" charging phase can be set.

## Network Drivers

Using the following registry settings, you can set up static parameters for network connections:

[HKEY_LOCAL_MACHINE\Comm\GSPI86861\Parms\TCPIP] – for Marvell GSPI8686 Wi-Fi

or

[HKEY_LOCAL_MACHINE\Comm\DM9CE1\Parms\TcpIp] – for DM9000 Ethernet

"EnableDHCP"=dword:0

"IpAddress"="10.1.1.44"

"Subnetmask"="255.255.255.0"

"DefaultGateway"="10.1.1.1"

"DNS"="212.150.48.169"

## Customizing System Power States

While using battery power, you can change the system power state transition timeouts using the following registry keys:

[HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Control\Power\Timeouts]

"BattSuspendTimeout"=dword:0

"BattBacklightTimeout"=dword:0

"BattUserIdle"=dword:0

If you need customized power behavior while running your application, consider the following recommendations:

To avoid device suspension, an application can send a keystroke using keybd_event(). Even sending an UP keystroke, you may still get a keyboard click, so use KEYEVENTF_SILENT to prevent the click from occurring. You should send a key that is currently not being used by the Windows CE operating system to eliminate any potential keyboard conflict. In the following example, an F4 UP keystroke is sent:

keybd_event(VK_F4, 0, KEYEVENTF_KEYUP | KEYEVENTF_SILENT, 0);

A better solution is to use the API SystemIdleTimerReset() from Coredll.dll. This does not disable the timeout, but resets the idle timer count back to zero. Since the minimum idle timer you can set is one minute, calling this API at intervals of less than one minute will prevent the device from entering suspend mode.

# Helpful Utilities

## RegEdit Utility

The image includes an embedded registry editor able to write/read/delete registry values.

Syntax:  reg.exe <(-)Registry Key> (h) <Value>

("-" before  Registry Key will remove the value, “h” for hex value).

## SetPowerState Utility

This is a command line utility, which can perform global system power state transitions.

Syntax:  SetPowerState.exe <State> <HexStateFlags> <HexOptions>

where <State> is system power state defined in the registry, e.g. Reboot.

optional <StateFlags> specify state flags, in hex, e.g. 200000 (POWER_STATE_SUSPEND).

optional <Options> specify options, in hex, e.g. 1000 (POWER_FORCE),

The most useful transitions are:

“SetPowerState reboot”	  – reboots the Exeda with the help of a watchdog

“SetPowerState suspend” – enters PXA27X deep sleep mode

Before power state transition, the registry is flushed to NAND.

## AutoLaunch Utility

Syntax: AutoLaunch.exe

The AutoLaunch utility extends the functionality of the “HKEY_LOCAL_MACHINE\Init” registry key by requiring the network stack to be configured with an IP address, before launching additional applications.  It also allows each application to have its own configurable delay before being launched.

The AutoLaunch monitors the following registry keys:

[HKEY_LOCAL_MACHINE\Startup]

“Process0” =”Application.exe param1 param2”

“Process0Delay”=dword:delay ;in milliseconds

[HKEY_LOCAL_MACHINE\Startup\ApplicationName]

“Process0” =”Application.exe param1 param2”

“Process0Delay”=dword:delay ;in milliseconds

Example of usage:

[HKEY_LOCAL_MACHINE\Startup]

"Process0” =”ceplayer.exe \windows\sound2.wav”

“Process0Delay”= dword:1388

or

[HKEY_LOCAL_MACHINE\Startup\ceplayer]

"Process0” =”ceplayer.exe \windows\sound2.wav”

“Process0Delay”= dword:1388

The above exampple will launch playback of sound2.wav 5 seconds after boot completion.

## SysConf Utility

This is a command line utility that allows advanced system analysis and debugging.

Read more about the Sysconf utility in \Docs\ Advanced System Analysis with SysConf Utility.doc

# Notes and Errata

- Battery measurement is not accurate during charging or when a USB-powered device is inserted into the USB OTG (primary) socket

- Camera stills resolution is 480x360 and not a full 2 mega pixel

- The Telit RIL driver doesn’t support modem power off

- The USB flash drive inserted in the USB OTG (primary) port should be reinserted after a Suspend/Resume cycle in order to be recognized

- When suspending during audio playback ,the device fails to play audio on resume

- No Bluetooth service when Wi-Fi is disabled  (will remain unchanged)