# CM-iAM BIOS UPDATING PROCESS

Revision Notes

| 15-10-2009 | - Initial public release |
| --- | --- |
| 27-12-2009 | - Added User CMOS defaults (“Save CMOS as defaults”) support - Added APCC(Atom preboot chipset configuration) support - Added Etherlink support - Updated microcode to version 26_18 |
| 01-04-2010 | - Added 16-02-2010 Bios Release 1.Added automatic detection for fitpc2i 2.LVDS related menu appears only on CM-iAM (hidden for fitpc2i) 3.Added 2GB BIOS version - Changed BIOS update procedure (p3 – devel34x.rom to biosx.rom) |

Updating using external USB

## Preparation Steps

1.Insert removable disk-on-key into USB slot ( ensure you don't have any other USB mass  storage devices connected).
2.Run ImUpdate.exe utility and select as "Target" desirable disk (by letter/size).
3.Choose 128MBDOS.img as "Source image".
4.Press "Start Restore" key to proceed and wait until completion. 
5.Reinsert bootable USB device wiht preinstalled image 
6.Copy to the root directory files SFlash16.exe and the BIOS image (biosx.rom-1045k image)

- To update BIOS
1.Insert USB disk and power on the system
2.Boot to DOS 
3.Type SFlash16.exe biosx.rom /Force
4.Wait until completion
5.Restart

Updating Using Ethernet Link

- Connect the  ethernet port (P4 on SB-iAM baseboard) to the LAN  (switch/hub) via a standard network cable.

- Ensure that no SATA disk is connected to the P35 connector

- The Host PC requires a TFTP server supporting TFTP options and large files (>32Mb). Recommended TFTP servers:

- “TFTPD32 by Ph.Jounin” TFTP server for a Windows based host (http://tftpd32.jounin.net/).

- TFTPD package for your distribution on a Linux based host

(for example: install tftp-server-0.34-1mdk on Mandrake).

Please note that TFTPD file names are case-sensitive; therefore, the file name typed in EtherLink must be in the same case as the file name of the BIOS image.

- Set "c:\bios_upd\" as the TFTP server base directory. For Windows tftpd3,2 choose “Settings” and browse for “Base directory”. Choose standard TFTP security.

- On the host  Enter BIOS[F2] , go to the “EXIT” submenu and select “Start EtherLink” .

- Configure EtherLink by pressing 1 (Configure) in the main menu. Carefully read "Etherlink.doc" to accomplish successful configuration.

- Ensure the BIOS image name (specified in line 5 of Etherlink menu /or line 6 of Config menu) is as in the c:\bios_upd folder (i.e., "bios.bin" – 1024k image).  If not, change the BIOS image name by pressing 6 in the Config menu.

- Return to the Etherlink main menu by pressing 0. Review the TFTP server parameters at the end of line 5.

- Start a secure BIOS update by pressing 5 (Download BIOS image) and then “Y”.  Do not interrupt the host PC and CM-iAM until the process is completed!

- Wait for the process’ completion. It takes about 30-40 sec.

- Return to the main menu by pressing any key. Press 0 to save the EtherLink configuration for future operations.

After Update

- Reboot the CM-iAM by pressing the RESET button. Verify that the BIOS revision date has changed, according to the revision date of the BIOS you installed. The BIOS date is specified in the bottom left corner of the BIOS configuration box displayed upon startup. If the BIOS date has not been changed - you probably missed something in the installation steps and are still using the previous version of BIOS. In this case, please repeat the installation steps correctly.