# EtherLink - Software Update Utility for CM-iAM

## Introduction

EtherLink enables fast binary transfers between a host computer and a target Computer-on-Module, through an Ethernet link. The network communication is based on the TFTP protocol.

EtherLink can write or read back contents of the NAND flash for the purpose of o/s package’s installation/duplication, as well as write/read BIOS and Configuration Block.

Host system: any computer with an Ethernet port and a TFTP server supporting large files (>32MB).

Target System: CM-iAM

## Installation

The EtherLink utility is an integrated part of BIOS and resides in the boot flash device.

The Host side requires installation of TFTP server supporting TFTP options and large files (>32MB).

Recommended TFTP servers:

- “tftpd32 by Ph.Jounin” TFTP server for a Windows based host (http://tftpd32.jounin.net/).

- tftpd package for your distribution on a Linux based host

(for example: install tftp-server-0.34-1mdk on Mandrake).

Please note that TFTPD file names are case-sensitive; therefore, the file name typed in EtherLink must be in the same case as the file name of the BIOS image.

## Activation

- Interactive activation through BIOS [F2] >>Exit>> Start EtherLink menu. Used for s/w installation and duplication.

## Usage

The EtherLink menus are self-explanatory.  The full menu tree of EtherLink is presented below. The value in brackets specifies the meaning of the parameter which should be provided and its default value.

- Configure

  - Use DHCP to obtain ip address for the device (Yes)

  - Set static ip address for the device (0.0.0.0)

  - Set subnet mask for the device (0.0.0.0)

  - Set ip address of the tftp server (server-ip-address)

  - Set NAND image file name (NAND-image-filename)

  - Set BIOS image file name (BIOS-image-filename)

  - Set configblock file name (configblock-image-filename)

  - Set NAND image start sector (00000000h)

  - Set NAND image end sector (ffffffffh)

1.A  Enable split (for nand size >2Gb)

- Download NAND image (from tftp://server-ip-address/NAND-image-filename - full image)

- Upload NAND image (to tftp://server-ip-address/NAND-image-filename - full image)

- Download BIOS image (from tftp://server-ip-address/BIOS-image-filename)

- Download configblock (from tftp://server-ip-address/BIOS-image-filename)

- Upload configblock (to tftp://server-ip-address/configblock-image-filename)

- Upload BIOS image (to tftp://server-ip-address/configblock-image-filename)

- Save configurations

- Quit EtherLink and reboot

The operation of the EtherLink includes the following steps:

- Setup network and remote host (Configure menu)

- The user is asked to use DHCP (automatic) settings or to specify network parameters manually. The relevant parameters are: target system IP address and subnet mask

NOTE: The utility is not intended to work with different subnets or to make any sort of name resolution; therefore, parameters such as default gateway and DNS server IP are not relevant.

- The user is asked for the TFTP server’s IP address and for an image filename for each supported image type (Menus 1.4 - 1.7).

- The partial NAND Flash image can be specified with the start and the end sector parameters.

For the download command, the remote file image will be updated on the NAND Flash starting from the start sector until the end sector or the end of the remote file.

For the upload command, the remote file image will be updated with NAND Flash data read from the start sector until the end sector or until the NAND Flash’s last sector (if the end sector is not specified - 0xffffffff)  (Menus 1.8 - 1.9).

- Accepting commands:

- Download NAND image - The contents of the remote image file from the TFTP server are written into the NAND Flash.

- Upload NAND image - The contents of the NAND Flash are read back into the remote image file on the TFTP server. For  files >2Gb “Enable split “ option should be enabled(default) – it will automatically splits image into 2 files adding *_01,*_02 extensions to the file name.”Enable split “ option is mutual exclusive with the NAND image start/end custom sector . Disable this option for 1Gb NAND flash or for files < than 2Gb.

- Download BIOS image (with confirmation) – The BIOS is updated with the contents of the remote BIOS image file from the TFTP server. The remote BIOS image is downloaded twice and only if the images are identical does the actual update take place. Moreover, an additional check for image size and contents is performed in order to ensure that the BIOS image is valid for the module (expected image size is 512 KB).Configuration block isn’t changed during this operation.

- Download configblock - The configblock is updated with the contents of the remote configblock image file on the TFTP server. The configblock contains BIOS factory defaults and EtherLink settings and APCC code. The update is performed in two phases like in the BIOS’ case (expected image size is 64 KB).

- Upload configblock - The contents of configblock are read back into the remote configblock image file on the TFTP server.

- Upload BIOS image - The BIOS is read back (including config block) into the remote BIOS image file on the TFTP server.

- Loading/Saving EtherLink configuration:

- The EtherLink performs an automatic load of configurations on activation. In the case that the configurations are not found or are corrupted, the following default configurations are used:

Use DHCP for automatic IP address acquires.

Remote TFTP server IP address – 00.00.00.00

Remote NAND image filename – disk.dat

Remote BIOS filename – bios.raw

Remote configblock filename – config.raw

NAND partial image start sector – 00h

NAND partial image end sector – ffffffffh

Enable split - Yes

- The EtherLink configurations can be saved into BIOS configblock for later use.

## Problems

Uploading the NAND Flash image (>32 Mb) to a remote TFTP server that doesn’t support large files (>32 MB) and options negotiation will yield unpredictable behavior.