U-Boot for EM-X270

Reference Guide

Table of Contents

1.	Revision Notes	3

2.	Overview	4

3.	Supported Features and Commands	5

3.1.	Standard U-Boot features	5

3.2.	Additional features	7

4.	Default Environment	8

5.	Boot Sequence Options	9

6.	Updating U-Boot	10

7.	Memory Map	13

# Revision Notes

| Date | Description |
| --- | --- |
| 20-Oct-2007 | - First release |

Please always check for a newer revision of this manual in CompuLab's website - http://www.compulab.co.il, following [Developer] >> [EM-X270] links.

# Overview

The EM-X270 uses the U-Boot boot-loader for low-level initialization and operating system loading.

U-Boot is an open source boot-loader available for a wide range of embedded processor architectures.

The purpose of this document is to provide a short description of the supported features and to summarize the settings unique to the EM-X270 port of U-Boot. Comprehensive documentation of the U-Boot project may be found at http://www.denx.de/wiki/view/DULG/UBoot.

# Supported Features and Commands

## Standard U-Boot features

The EM-X270 port of U-Boot supports the following standard U-Boot features:

- Information commands

bdinfo − print Board Info structure

coninfo − print console devices and information

flinfo − print FLASH memory information

help − print online help

- Memory commands

base − print or set address offset

crc32 − checksum calculation

cmp − memory compare

cp − memory copy

md − memory display

mm − memory modify (auto incrementing)

mtest − simple RAM test

mw − memory write (fill)

nm − memory modify (constant address)

loop − infinite loop on address range

- Flash memory commands

cp − memory copy

flinfo − print FLASH memory information

erase − erase FLASH memory

protect − enable or disable FLASH write protection

- Execution control commands

autoscr − run script from memory

bootm − boot application image from memory

go − start application at address 'addr'

loop – infinite loop on address range

exit – exit script

- Download commands

bootp − boot image via network using BOOTP/TFTP protocol

dhcp − invoke DHCP client to obtain IP/boot params

loadb − load binary file over serial line (kermit mode)

loads − load S−Record file over serial line

rarpboot− boot image via network using RARP/TFTP protocol

tftpboot− boot image via network using TFTP protocol

nfs – boot image via network using NFS protocol

ping – send ICMP ECHO REQUEST to network host

- Environment variables commands

printenv− print environment variables

saveenv − save environment variables to persistent storage

setenv − set environment variables

run − run commands in an environment variable

bootd − boot default, i.e., run 'bootcmd'

- File system support commands

fatinfo – print information about a FAT file system

fatload – load binary file from a FAT file system

fatls – list files in a FAT file system directory (default /)

fsinfo – print information about JFFS2 file system

fsload – load binary file from a JFFS2 file system image

ls – list files in a JFFS2 file system directory (default /)

- NAND flash support

nand – NAND sub-system control

nboot – boot from NAND device

- MMC support

mmcinit – detect MMC card

- USB support

usb – USB sub-system control

usbboot – boot from USB device

- Bitmap support

bmp – manipulate BMP image data

- Miscellaneous commands

echo − echo arguments to console

reset − Perform RESET of the CPU

sleep − delay execution for some time

version − print monitor version

test – minimal test, like /bin/bash

? − alias for 'help'

To get more information about a specific command, type “help [command]” in the

U-Boot command line.

Please refer to the U-Boot project documentation for detailed descriptions of the features listed above.

## Additional features

The following features are specific to the EM-X270 port of U-Boot:

- CPU frequency scaling

cpufreq:  with no arguments - display the current frequency setting

with a numeric argument - set CPU frequency; (allowed frequency values : 104/208/312/520)

- Hardware configuration information

hwinfo:  print device hardware configuration information

# Default Environment

U-Boot features an extensive default environment, for boot sequence control and hardware configuration.

| Variable | Default setting | Description |
| --- | --- | --- |
| bootcmd |  | This variable defines a command string that is automatically executed when the initial countdown is not interrupted. |
| bootdelay | 3 sec | Delay before automatic boot starts |
| boot_mode | uboot | Boot option setting. See section 5 for details. |
| bootargs |  | The contents of this variable are passed to the Linux kernel as boot arguments |
| stdin | serial | Standard input stream |
| stdout | serial | Standard output stream |
| stderr | serial | Standard error stream |
| baudrate | 115200 | Serial port baud rate |
| ethaddr | set at production time | Ethernet MAC address for the EB-X270 extender board ethernet interface |
| ipaddr | - | IP address; needed for tftp command |
| serverip | - | Server address; needed for tftp command |
| autoload | no | Determines whether the “dhcp”, “bootp” and “rarpb” commands will automatically download an image. |
| splashimage | 0x000B0000 | Address of a bitmap image used as splash-screen. |
| usb_mode | slave | USB port operation mode. The only values allowed are: “slave” or “host”. Must be set before “usb start” command is issued. |
| cpu_freq | maximum supported by CPU | CPU frequency. Used by “cpufreq” command to store the frequency setting. |
| mem_zero | on | Determines whether memory zeroing is performed. Allowed values: “on”, “off”. Set to “off” for shorter boot time. |

# Boot Sequence Options

Boot Options

The U-Boot boot-loader supports several basic boot options.

The environment variable “boot_mode” determines which boot sequence is executed.

The variable may be set to the following values:

- “uboot” – the EM-X270 boots up to the U-boot command prompt.

- “linux” – the default Linux boot from NAND sequence is executed. Please see the “Linux Manual” for details.

- “wince” – the default WinCE boot from NAND sequence is executed. Please see the “WinCE Manual” for details.

- any other value – the command stored in the “bootcmd” variable is executed.

Bringing to Default State

Pressing and holding ‘Ctrl D’ during the boot process will restore the U-Boot settings to the default factory values. Use this if the environment has been corrupted.

O/S Update Boot Sequence

Pressing the ‘Sleep/Resume’ button for more than four seconds during the boot process will put the system into the O/S update boot sequence. The boot-loader will attempt to detect an external storage device (USB or MMC) and search for updated image files. If no files are detected, the system will boot up to the U-Boot command prompt. This boot mode allows easy and fast software updates without connecting a PC host.

Please refer to O/S manuals for specific update procedures.

NAND Flash System

The EM-X270 port of U-Boot supports two separate NAND flash drivers.

- The MTD flash driver is used with the Linux O/S.

- The Compulab NAND flash driver is used with the WinCE O/S.

U-Boot will load the appropriate driver according to the “boot_mode” environment variable. Additionally, the “nand_mode” environment variable may be used to set the NAND driver mode explicitly. Setting “nand_mode” to “on” will cause U-Boot to load the MTD driver. If “nand_mode” is set to “off” the MTD driver will not be loaded. See the “WinCE manual” for further details.

# Updating U-Boot

NOTE: Do not interrupt the process or shut down the system during update. Doing so can result in a disabled EM-X270 board that will need to be shipped to CompuLab for reprogramming.

- Update through serial port (preferred method)

- Start terminal emulation program on your PC using the serial port to which the supplied serial cable is connected. Set baud rate to 115200 bps, 8 bit per frame, 1 stop bit, no parity, no flow control.

- Turn on the EM-X270 board. U-Boot will start and you will get a command prompt.

- Type ‘loadb a0010000’.

- In a terminal emulation program, send the U-Boot image file using kermit mode. 
For example, in HyperTerminal, use “Transfer >> Send File”. A “Send File" dialog will appear. In the protocol combo box, choose Kermit. Fill in the U-Boot image filename and press the “Send” button.

- Type ‘protect off 0 +50000’.

- Type ‘erase 0 +50000’.

- Type ‘cp.b a0010000 0 50000’.

- Reboot the EM-X270.

- Update through USB and Disk-on-Key

- A USB disk-on-key storage device is need for this procedure.

- On the host PC, copy the "u-boot-x.x.x.bin" file to the disk-on-key. Rename the file “u-boot.bin”.

- Connect the disk-on-key to the EM-X270 using the supplied “USB-A receptacle to Mini-USB A male” cable.

- Start a terminal emulation program on your PC using the serial port to which the supplied serial cable is connected. Set baud rate to 115200 bps, 8 bit per frame, 1 stop bit, no parity, no flow control.

- Turn on the EM-X270. U-Boot will start and you will get a command prompt.

- Type ‘setenv usb_mode host’.

- Type ‘usb start’. U-Boot will detect the USB storage device.

- Type ‘fatload usb 0 a0010000 u-boot.bin’.

- Type ‘protect off 0 +50000’.

- Type ‘erase 0 +50000’.

- Type ‘cp.b a0010000 0 50000’.

- Type ‘usb stop’.

- Reboot the EM-X270.

- Update through LAN / TFTP

- The EB-X270 extender board is needed for this procedure.

- Install TFTP service (or server) on the host PC. The TFTP server tested by Compulab is TFTPD32, http://tftpd32.jounin.net, it is also included in the Linux and WinCE distribution packages.

- Rename the "u-boot-x.x.x.bin" file “u-boot.bin” and copy it to the TFTP directory.

- Start a terminal emulation program on your PC using the serial port to which the supplied serial cable is connected. Set baud rate to 115200 bps, 8 bit per frame, 1 stop bit, no parity, no flow control.

- Turn on the EM-X270. U-Boot will start and you will get a command prompt.

- Type ‘printenv’ in order to check the current IP settings. Modify them if needed using the ‘setenv ipaddr’ and ‘setenv serverip’ commands. Or, use U-Boot’s DHCP capability to automatically set the IP address.

- Type ‘tftp a0010000 u-boot.bin’

- Type ‘protect off 0 +50000’.

- Type ‘erase 0 +50000’.

- Type ‘cp.b a0010000 0 50000’.

- Reboot the EM-X270.

# Memory Map

NOR Flash

NOTE: The NOR flash memory is used to store the U-Boot image, the U-Boot environment and EM-X270 setup data for wireless interfaces. Corruption of this data can result in a disabled EM-X270 board that will need to be shipped to CompuLab for reprogramming.

| Memory Address range | Block Range | Size | Usage |
| --- | --- | --- | --- |
| 0x00000000 – 0x0004FFFF | 1 - 5 | 320KB | U-Boot bootloader |
| 0x00050000 – 0x0005FFFF | 6 | 64KB | U-Boot environment |
| 0x00060000 – 0x0006FFFF | 7 | 64KB | Setup block |
| 0x00070000 – 0x000AFFFF | 8 - 11 | 256KB | Utilities and applications |
| 0x000B0000 – 0x000FFFFF | 12 - 16 | 320KB | Splash-screen |

SDRAM Memory

SDRAM map: SDRAM starts at 0xA0000000; maximum is 0xA8000000 (128MB).

U-Boot system:				0xA7800000……………….. SDRAM top

U-Boot operates with a virtual memory subsystem turned on, using one-to-one virtual to physical address mapping.