Filesystem Protection

X270 Linux package uses EXT3 file system. In event of crash, only recent changes are lost, while filesystem is not corrupted. Tests conducted in CompuLab shown that EXT3 filesystem is very tolerant to power failure events.  However methods specified below provide even more robust protection.

Write-Protected Partition

Divide your filesystem into two (or more) logical partitions. Define one (or more) partitions as read-only. Operating system would not issue requests for modifying read-only partition contents and consequently Flash Disk Driver would not alter its used blocks. The size of logical partition should be defined in increments of 128 KB, to match NAND flash block size.

Notes:

- It is recommended to install partitioned Linux on clean NAND flash, which was previously formatted under ARMmon, and also install the contents of read-only partition before any further writes to the file system. Although protection method does not dependent on this requirement, the validity proof was performed only for that case.

- Flash Disk Driver is not aware of logical partition boundaries, as it works with physical blocks. However, the absence of write requests from operating system assures that blocks would not be modified, if they are not empty. On the other hand, the empty (i.e. unused) blocks remain belonging to Flash Driver's depository, and therefore could be modified by Flash Driver, even if they were defined as a part of read-only logical partition.

This note is only for clarification, the statement that read-only partition is protected remains true.

Early detection of power fail event

Connect a 2F (Farad) supercap on the power supply entry to keep the processor functional for a short period after power failure. Provide power fail indication through a general purpose input. When the application needs to update a file, it should act as follows:

- Check power fail indication. Don't proceed if true.

- Open the file for writing

- Seek to the file's end if necessary

- Write to the file, limiting data length to less than 128KB

- Close the file

Supercap will hold the power required for successful completion of steps 2 to 5.

While calculating the required capacitance, you should assume that nominal 3.3V supply voltage is allowed to drop down to 3.1V, but not less. A 128KB (max) file write operation should be completed before the voltage drops below the critical threshold.  A 128KB write takes less than 0.3 sec. For example, assuming 1A current consumption, 0.3 sec until file closing, and an additional 0.2 sec margin, the capacitor should be: (1A * 0.5 sec) / 0.2V = 2.5 farad. Supercaps of such a capacity are coin-sized.

Run from RAM Disk

Keep Linux file system on Flash Disk in read-only, preferably compressed form. On start, open RAM Disk and expand Linux to RAM disk as active file system. Then all temporary writes to file system, which Linux must perform, will be on RAM disk. This method is fully tolerable to power fails, because clean copy of Linux file system is created in RAM disk on every powerup.

Use two flash disks devices

CM-X270 has two separate Flash Disk devices on board - NOR and NAND.

You can use one flash disk as read-only, and another flash disk to store files, with optional additional protection means as mentioned above.