Win CE 5.0 for CM-X270

Installation & Feature Guide

This version of Windows CE is intended for evaluation purposes only. It may be used for up to 30 days. After this period, for continued use, the user must buy an appropriate Windows CE license.

# Release Notes

| 15-Nov-2005 | - First Release of Win CE 5.0 for CM-X270 |
| --- | --- |
| 03-Dec-2005 | - Added explanation of display support |
| 25-Dec-2005 | - Added support for 2700G Multimedia Accelerator - Added support for 3rd Host USB port (of IT8152) - Added support for 8 / 16 bit color support for PXA270 graphics controller. Previous version supported only 18/24-bit color which slowed the processor. - Added RTC support |
| 20-Mar-2006 | - Added support for Suspend/Resume (entering deep sleep mode) - Added custom panel support for 2700G Multimedia Accelerator - Added System activity indicator (red led is on when active) - NAND Flash Driver updated to rev 3.05 (solved some stability issues) - Fixed a number of USB slave connection issues - Fixed the COM ports initialization order (COM4 works with rev 1.2) - Fixed system Timer model |
| 27-Aug-2006 | - The BSP was rewritten using Intel's new MAINSTONE-III for PXA270 - Added support for 128MB RAM - Added built in Wi-Fi support for CM-X270L - Added Hard disk support. - Added GPIO driver - Added I2C driver - Fixed NOR flash driver to avoid interference with splash screen support |
| 15-Oct-2006 | - The BSP build system was improved (see BSP documentation) - Added support for Rotation on PXA270 LCS - Added support for SDIO - Fixed the KITL issue (see BSP.doc) - Added support for software reboot - Registry flush policy improved - Added a flush mechanism to the NAND Flash Driver - Added automatic disable of the WI-FI driver on the CM-X270W - Added an automatic COM port switch to support both the CM-X270W and CM-X270L - Added a reg.exe utility to read/write registry values - Added a SetPowerState.exe utility to switch between different power states |
| 16-Nov-2006 | - WinCE COM port order documentation and driver were updated. - Added some x270_CE5 project default components: - USB Mass Storage - .NET 2 - Storage Manager Control Panel - Fixed the pxa270_lcd driver resume issue. - Fixed the pxa270_lcd to allow Debug compilation. |
| 14-Dec-2006 | - Added debug output support on COM-A via registry. Read “Analyzing the WinCE image problems” - Added Flash Driver guide for asynchronous power off events. Read “Asynchronous Power Off Events and NAND block corruption “ - Added note about removing some SB-X270 capacitors for resolving touch screen problem. See touch screen section. - Added a note about sound recording. See “Audio Driver” section. - Fixed audio driver suspend / resume issue. - Fixed audio driver to properly support Crystal codec ("A" option). - Fixed RTC accuracy problem. - Default image is now supported on all partial CM-X270 configurations. - Fixed Wi-Fi vs. Audio DMA conflict. - Added Suspend / Resume capability for Wi-Fi driver. - Added support for resume on GPIO1 falling edge transaction. - Fixed 2700G suspend / resume exceptions. - Fixed registry flush timeout to be 30 sec. |
| 27-Dec-2006 | - Fixed Audio driver recording quality issue. - Fixed AC97 link reset mechanism that caused audio driver to fail during the initialization. - Added support for full power management to the NAND Flash Driver. - Added Flash Driver guide for asynchronous power off events. Read “Asynchronous Power Off Events and NAND block corruption “ - Added support for PCCARD timings configuration via registry. - PCCARD timings changed to improve stability. Added a note about the system instability that can be caused by too fast PCCARD timings. - Fixed Sleep mode power consumption problem. |
| 31-Jan-2006 | - Fixed Davicom driver to support MAC address specified by ARMmon. - Fixed SD card issues: detection, write protect, suspend/resume. - Fixed PCMCIA insertion system lock problem. |
| 19-Apr-2007 | - Updated System Timer driver to improve system stability, according to Microsoft’s recommendation. - Fixed rare system freezes during cold boot, caused by garbage in memory left after hardware reset with "fastboot on". - Added host support for USB3 port (see USB Support section). See USB3 Dual Function section. - Fixed USB detection during the boot issue. - Added an ImageUpdater utility that allows update of the WinCE image in the NAND Flash from a remote drive or any SD, USB or PCMCIA storage cards. - Added abilities to control Win CE functions from ARMmon, by passing parameter during the boot: - Enable debug messages for diagnostics - Enable / disable of any driver - LCCR values, for LCD evaluation - Static IP, for telnet session establishment in headless configurations |
| 12-July-2007 | - Added keypad driver. - Updated GPIO driver. - Added sample SPI driver. - Fixed RTC bug. |
| 18-Oct-2007 | - Changed NAND flash driver, to be compatible with new NAND specs. - Added SPI driver. - Comprehensive modification to I2C driver. - Fixed bug in keypad driver. |
| 06-Nov-2008 | - Added support for GSPI8686 WiFi. - Fixed a bug that caused the system to freeze. - Add functionality to SPI driver. - Fixed GPIO driver bug. - Fixed I2C bug. - Fixed keypad bug. - Improved DMA handling mechanism. |
| 23-July-2009 | - Modified Touch-Screen driver (now supports both Wolfson & UCB codec). - Added WiFi support for CM-X270W rev 1.4. - Added BlueTooth support for CM-X270W rev 1.4. - Sysconf utility added. - Added support to Wolfson codec in the WAVDEV2 driver. - Fixed a bug that caused a few seconds gap (black screen) between the splash-screen and the WinCE screen. - Fixed a NAND flash driver bug. |

# Features

## Supported Features

- NAND and NOR Flash Disks

- LCD Panels & CRT Displays

- Power Management

- PXA270 graphics using 8/16/18 bit color

- 2700G graphics, including 2D acceleration, MPEG, Media Player and panel configuration

- Network, with Static IP address or DHCP

- USB Slave

- USB Host ports of PXA270 and ITE8251

- Audio

- Touchscreen

- Serial Ports (COM1 – COM4)

- USB Keyboard and Mouse

- PCMCIA

- Hard disk

- RTC

- Wi-Fi driver

- GPIO stream driver

- I2C stream driver

A run time component list is found at the end of this document.

## Features Not Yet Supported

- CardBus (32-bit)

- 3D acceleration features of 2700G

- VIP / Quick capture interface

# Installing the Win CE Image on the CM-X270

The following preparatory steps should to be taken:

- Connect the CRT or Flat panel monitor, USB keyboard and USB mouse to the SB-X270 baseboard.

- Connect the SB-X270’s COM-A (RJ-11 type connector on the front panel) to the serial port of the host computer, using the serial cable provided with the Evaluation Kit.

- Create the "disk.bin" image file using the "disk_bin_generator" program provided in this archive. For more information regarding the "disk.bin" file, see the "Image File" section below.

- Connect the CM-X270’s Ethernet port (P25 on the SB-X270's front panel) to the network which your development workstation is connected.

- Ensure that you are using up-to-date ARMmon:

  - Download the ARMmon package available following the [Developer] >> [CM-X270] >> [ARMmon - image and documentation] links in CompuLab's website.

  - Check the ARMmon date of your card (reported on startup) against the latest revision note date specified in "Revision Notes" of the "ARMmon Manual" document.

    - If your ARMmon date matches the date of the latest revision - no update is required.

    - If your ARMmon date is earlier than the latest revision, then check your ARMmon date against the latest revision date of the Win CE package, as specified in this document ("Installation & Features"). If your ARMmon is older than the Win CE package - you need to update.

In other words, you should ensure that you are using either the latest version of ARMmon or at least a version not older than the Win CE package. The described procedure ensures that you will use up-to-date ARMmon, while still avoiding unnecessary updates.

Note that Evaluation Kits and cards from stock are not necessarily shipped with the latest version of ARMmon. Win CE installation won't run properly with old ARMmon.

There are several installation methods. The most preferred methods utilize the Ethernet. The other method is to use UsbMLink utility.

- Preferred Installation using Ethernet

This method is the fastest and most flexible.

It requires a running TFTP server on a host workstation running Windows 2000 or XP.

The recommended TFTPD32 server executable is supplied with this Win CE package, located in the “TFTP Server” directory.

There are two options of installation using Ethernet:

Option 1:

Store the image and partition table (disk.bin) in the NAND Flash and boot from it.  The boot process involves downloading from the NAND flash to the SDRAM.

- Using ARMmon, format the NAND Flash:

nand flash format

- Reboot the CM-X270.

- Copy the previously prepared disk.bin to the TFTP server repository.

- Using ARMmon, download the disk.bin image to NAND flash:

nand write disk.bin <IP Address of TFTP>

- Follow the "After Installation" section below.

Option 2:

Download the nk.bin raw image to RAM though the network, and execute it. This procedure is well suited to the experimental stage, when image contents are changed frequently. The procedure has to be repeated after every reboot.

- Copy the nk.bin file to the TFTP server repository.

- At the ARMonitor prompt, type the following command:

download wince tftp nk.bin <IP Address of TFTP server>

- When image downloading is complete, type bootos.

- Notes:

- The IP address of the TFTP server is displayed in your TFTPD32 GUI.

- If you use firewall software on your host PC, ensure that you unblock the TFTP Server port (defaults to port=69).

- Installation using USB link

- This method uses a USB slave port of the CM-X270 (see the SB-X270 user manual for USB3 port location and comments). It requires an ActiveSync installed on your Windows XP-based host PC.

- Installation process:

- Download the UsbMLink utility from [Developer]>>[CM-X270]>>[UsbMlink] in CompuLab's website. Following its documentation, install and activate the UsbMLink.

- Rename the "disk.bin" file (the filesystem image you created during the preparatory steps) to "nand.in". Using UsbMLink_GUI, run [NAND Update]. It will take about 15 minutes.

- Disconnect the USB from the target and follow the "After Installation" section below.

- After Installation

- Reset the CM-X270.

- To start Win CE, type "bootos" at the ARMmon prompt. If you prefer Win CE to start automatically, type “setboot os”, “save” and then “bootos”. (In this case, if you want to return to the ARMmon prompt, press Ctrl+C during reset)

Remark: “bootos” performs a search for installed operating systems, and boots the first one it finds.  A detailed explanation is available in the ARMonitor documentation. The boot order is: check for Linux in RAM, check for Win CE in RAM, check for a Linux kernel in NOR and check for Win CE in NAND. If Linux is installed in NOR, reboot the core, enter ARMmon and perform “flash kernel” to delete the operating system from NOR.  Then “bootos” will succeed in finding Win CE on NAND.

See the “WinCE Demo Image Customization” section for details of the command line parameters for the “bootos” command.

# Image Configuration

The image available from CompuLab's website was built using a Platform Builder based on a custom configuration. It contains most of the components provided by Microsoft.

The main components included in the image are:

- Network support

- PCMCIA support

- Internet Explorer

- File system Explorer

- Media Player

- Command Prompt

- Help Engine

- Control Panel

- USB Storage

- IDE support

- WLAN support

- SD Memory support

- ActiveSync Communication Starter.

- Remote Display

- Servers (telnet, ftp ,file server,…)

- Useful utilities (reg.exe, SetPowerState.exe)

The size of this Windows CE image is relatively large, 25 MB, because many components are included.  Typical Windows CE images with a specific purpose are much smaller and in most cases don't exceed 5-10 MB. They therefore require less NAND flash and boot faster.

# Dynamic Registry Configuration

The provided image utilizes a Hive Based Registry, which means that whatever changes are made to it during platform operation will be stored on NAND and retained after reboot.

There are two ways to modify the registry: with programs (like installers) from the CE operating system itself or through the Remote Registry Editor.

# Running the User Program from Flash on Start-up

- Copy your application program to NAND.

- Create and copy to the NAND a file named “StartUp.bat”.  This file should contain the command to launch an application program or any windows component.
For example: "\Windows\activesynclauncher.exe" /serial (or /serial3 on CM-X270L)
is a valid command to launch an ActiveSync application on serial port 1.

- The “MainStartUp.bat” file that is built into the image will start the StartUp.bat file from the NAND/NOR.

# Remote Tools

Remote Tools allow you to monitor your target right from your desktop development environment. You can edit the target registry, view the processes that are running, take a snapshot of the target's screen, etc.

If you want to use Remote Tools in a Release version, you have to add the “Platform Manager” component in the Platform Builder.

This includes the three media capable of communicating with the target:

Manual server

ActiveSync

KITL (requires Platform Builder)

# Manual Server over TCP/IP

If you are working with a release version and manual server, a dialog box will come up to verify that certain files exist on the target side:

Select and copy the CEMGRC.EXE line but do not click the OK button yet. Open a telnet session to the target and paste the command line to the telnet session as shown in this picture:

Press ENTER so the target starts listening for connections from Remote Tools.

By pressing OK on the host, the remote communication setup will be completed.

NOTE: In order to start a telnet session, you need the IP address of the target, which can be obtained by running the ipconfig utility from the command shell on the target or by forcing the operating system to use the IP address configured in ARMmon (see the “WinCE Demo Image Customization” section for details).

# ActiveSync

The Windows CE image provided for the CM-X270 includes all the files necessary to establish a connection with a host computer. To do this, ActiveSync should be installed on the host computer. It may be downloaded from Microsoft's website (http://www.microsoft.com/windowsmobile/downloads/activesync42.mspx)

ActiveSync establishes a preliminary connection through a COM or USB port. However, from the moment a connection is established, ActiveSync will then use a network connection if one is available. It is important to remember that even when a network connection is used, the serial or USB cable should not be disconnected, as doing so will break the connection.

The following settings should be applied in the Connection Settings dialog box:

The simplest way to establish a connection between the target (CM-X270) and the host (ActiveSync) automatically is by plugging a USB cable into the USB slave port of the CM-X270. (The image is pre-configured for USB synchronization.)

If you want to start synchronization via a COM port, you should double-click on the desktop shortcut “ActiveSync on COM1”. This will start the ActiveSyncLauncher.exe application.  Please read the description of this program below.

In order to establish a connection between the target (CM-X270) and the host (ActiveSync) manually, the following steps are required:

On Host (workstation)

- If you wish to use USB for A.S., some preparations are required on the workstation:

  - Disconnect the USB cable between the CM-X270 and the workstation.

  - Install ActiveSync 4.1 (or later) on the workstation.

On Target (CM-X270)

- A connection entry should be created in “Networking and Dial-up connections”.
To do this, open “Start >> Settings >> Network and Dial-up connections”, then double click on "Make New Connection". The name of the new connection is not important. The connection type should be "Direct Connection". Press “Next” and select the device to be used. Typically the device will be "Serial Cable on COM1:” which represents COM1, or “USB: ”. Press “Finish” to finalize the connection’s creation. Close the “Network and Dial-up connections” window.

- This connection should be defined as the default connection for synchronization.
To do so, enter the “Control Panel” and double click on the "PC Connection" icon. Press the “Change” button and select the connection name that was defined in step 2. Close the “Control Panel” window. (This will update the registry settings).

- When USB is chosen as the connection transport, plug a USB cable between the CM-X270 and the host workstation. The connection will be established automatically. 
During a first time connection, be patient while Win2000 installs a new USB device.

- When establishing a connection over serial, on the A.S. menu, click “Get Connected….”.

- While A.S. is trying to establish a connection, do the following (on CM-X270): Open “Start >> Settings >> Network and Dial-up connections”, then double click on "My Connection". The connection should be established.

This procedure should be followed each time an ActiveSync connection is required. In order to simplify this procedure, a utility to perform these steps was written. The utility is called ActiveSyncLauncher (sources are provided in the DemoApps directory) and can operate with COM ports only (not relevant for USB!). In the command line, ActiveSyncLauncher receives the name of the device to be used to connect to ActiveSync. If nothing is specified, ActiveSyncLauncher assumes that the connection is via COM1.

For convenience, the icon is provided on the desktop. Double clicking on "ActiveSync on COM1"

Launches ActiveSyncLauncher.exe.

ActiveSync requires full modem control line support from the serial port. Thus, the only serial port suitable for ActiveSync is COM1.

# USB Support

A host USB driver is included in the Win CE image. However, the host USB driver alone is not sufficient for peripheral interfacing.  You also need a peripheral Device Driver, which should be obtained from the peripheral's manufacturer. The sample image includes all USB device drivers provided by Microsoft: keyboard, mouse, USB storage, smart card reader and USB printer driver. For any other peripheral support, please check with the peripheral’s manufacturer for device driver availability.

Some brands of USB keyboards may not initialize correctly, if they were connected to the USB during board initialization (booting of OS). Disconnecting or reconnecting the keyboard usually solves the problem.

After SUSPEND, it may take a few seconds for the USB keyboard and mouse to become operational. This delay does not interfere with the execution of background tasks.

The PXA270 USB host does not operate properly with the USB hub; specifically, after Suspend/Resume, the hub must be reinserted.

The USB3 port of the CM-X270 can be configured as a slave (default) or as a host. Selection of host or slave mode is performed by ARMmon and should be accompanied by proper jumpers settings.

To change the USB3 configuration to host:

- In ARMmon, type:  “usb3mode host”, and “save”.

- Power off the SB-X270 and shorten the pin10 with pin11 on P15 by a jumper.

To change the USB3 configuration back to slave:

- In ARMmon, type:  “usb3mode slave” and “save”.

- Power off the SB-X270 and remove the jumper (if any) from pin10, pin11 on P15.

# Ethernet

The sample Windows CE image is configured to use DHCP to get network settings for both network adapters. The network settings can be changed from the Windows CE Control Panel.

Debugging over KITL (Ethernet) is terminated by a Suspend/Resume sequence. If the debugger was active before Suspend/Resume, the platform will lose the debugging connection and hang.

# Wi-Fi

The WinCE release includes a driver for the Philips BGW200 WiFi controller integrated into the CM-X270L. The controller communicates with the CPU through an SPI interface.

The network adapter setting can be specified in the following registry key:

[HKEY_LOCAL_MACHINE\Comm\WLAN1\Parms\TCPIP].

The run-time image is provided with static IP, which possibly will not match your network settings. You can change the IP using the above-mentioned registry key, or alternatively set-up the system for DHCP.

The user can maintain wireless adapter (“WLAN1”) state with ndisconfig.exe (start from cmd) utility or Enable / Disable it in “Network Connections” control panel. See ndisconfig help in Platform Builder.

# PXA270 built-in LCD controller

Selection between the PXA270 LCD controller and the optional 2700G Multimedia Accelerator is performed by ARMmon and can be controlled by the "setgraphics" command.

(The actual selection activates the regmod.dll driver that starts before GWES).

Default display resolution is 640 x 480 and can be changed by rebuilding the image with different registry settings.  The default settings are directly compatible with VGA CRT displays and certain types of TFT LCD panels, such as Toshiba’s LTM04C380K.

An example of display registry settings (available in "display_pxa270.inc" in the BSP):

[HKEY_LOCAL_MACHINE\Drivers\Display\PXA27x\Config]

"DisplayDll"="pxa27x_lcd.dll"

"DefaultDisplayType"="TFT"

"CxScreen"=dword:280;

"CyScreen"=dword:1E0;

"Bpp"=dword:10 ;16 bpp

"Rotation"=dword:0

Where “DefaultDisplayType” can be “TFT” or “STN” and selects default timings for current resolution and display time.

The “CxScreen” and “CyScreen” specify the display resolution.

Rotation

Portrait/Landscape rotation can be achieved by using “Rotation”=dword:1.

(The actual rotation is performed by another driver pxa255_lcd.dll and it is automatically selected on boot time with regmod.dll)

The pxa270_lcd.dll can perform 180-degree rotation with the following registry key:

[HKEY_LOCAL_MACHINE\system\gdi\rotation]

"Angle"=dword:0 or dword:B4

(Note: "dword:" values are in hexadecimal. Changes in the registry will take effect after reboot )

Bpp (Bits Per Pixel) settings

The current version of pxa270_lcd.dll supports only 16 bit colors.

Customizing LCD timings

The PXA270 LCD timings can be customized with the following registers:

"LCCR0"=dword:

"LCCR1"=dword:

"LCCR2"=dword:

"LCCR3"=dword:

"LCCR4"=dword:

These values supersede the defaults established by the “DefualtDisplayType”.

Usually the user obtains these values from CompuLab and is then required to update the registry.

The simplest way to do so is to establish a Telnet session to the device and then update the LCCRX values using the reg.exe utility provided with the demo image.

For example, type “reg lccr0 <value> to update the LCCR0 register (see reg.exe help).

Another way to set up the LCCR values for the first time is to use the LCCR values from ARMmon and force the WinCE image to update the registry with these values.

From ARMmon, type “bootos “SETLCCR””. See the “WinCE Demo Image Customization” section for details.

Many other displays are supported. A more detailed list is available following the

[Developer] >> [X270] >> [Verified Panels List] links in CompuLab's website. Support of additional panels can be obtained using CompuLab's panel adaptation service.

## 2700G Multimedia Accelerator

Selection between the 2700G Multimedia Accelerator and the PXA270 LCD controller is performed by ARMmon and can be controlled by the "setgraphics" command.

The default display resolution is 640 x 480 and can be changed by rebuilding the image with different registry settings. The default settings are directly compatible with VGA CRT displays and certain types of TFT LCD panels, such as Toshiba’s LTM04C380K.

A example of some display registry settings (available in the "display_2700G.inc" file in the BSP):

[HKEY_LOCAL_MACHINE\Drivers\Display\PowerVR]

"Width"=dword:280 ;400;

"Height"=dword:1E0;300;

"BitsPerPixel"=dword:10

"Panel"=dword:00000002; 2 – VGA

Using the following registry key, the rotation angle can be specified:

[HKEY_LOCAL_MACHINE\system\gdi\rotation]

"Angle"=dword:0

Support of the following displays/panels is pre-built into the run-time image:

| Mfg | Type | "Panel" selection code |
| --- | --- | --- |
| (any) | CRT display | “Panel” 2, supports variable resolutions: 320x240, 640x480, 800x600, 1024x768, 1280x1024 Using 60 Hz refresh rate |
| SHARP | LQ057Q3DC02 | “Panel” 3, 320x240 |
| SHARP | LQ022B2DB02T | “Panel” 4 |
| TOSHIBA | LTM04C380S | “Panel” 5, 640x480 |
| SHARP | LLT1820H | “Panel” 6, 1280x1024 |
| SHARP | LS047D | “Panel” 7 |
| EPSON | LD22002TB301 | “Panel” 8 |
| (any) | USER DEFINED | “Panel” 9 |

The complete list of supported timings and resolutions can be found in the

C:\WINCE500\PLATFORM\CM_X270_BSP\SRC\DRIVERS\marathon\wince\pdp\common\ pdptimings.cpp file.

If you want to support a custom panel, you should choose “Panel =9” and read the “Setup Custom Panel for 2700G.doc” document.

# Touch screen

A driver for resistive touch screens is available in the provided package. The CM-X270 must contain the "AT" option to provide touch screen support.

Note: Some touch screens are very sensitive to electro-magnetic interference. The driver filters out sequences of samples that have large variance. Therefore, in a noisy environment, the user will find the touch screen unresponsive and sluggish. For example, in a clean environment, the driver provides 40 samples per second, whereas in a noisy environment (e.g., the LCD back light inverter has been turned on), only 20 samples per second are accepted.

On the CM-X270 without AC97 codec (i.e., without the "A" or "AT" options), it is necessary to rebuild the image with the "Exclude from build and Image" option for both Audio and Touch controller drivers. If the drivers are not excluded, the CM-X270 without AC97 codec will not be able to enter sleep mode.

NOTE:

Touch Screen drivers does not work properly when the following capacitors are assembled on SB-X270: C140,C143,C150,C151. In newer versions of SB-X270 these capacitors are not assembled.

PCMCIA

The PCMCIA timings can have critical impact on system stability.

Too fast timings can cause system instability and too slow timings can cause LCD flicks and slow performance of PCMCIA device.

The Fine Tuning of the timings should be performed for the specific PCMCIA card that is used in the customer system. The following registry settings allow the control of PCMCIA timings:

IO Timings

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\MCIO0_SET

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\MCIO0_ASST

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\MCIO0_HOLD

Attribute Timings

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\MCATT0_SET

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\MCATT0_ASST

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\MCATT0_HOLD

Memory timings

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\MCMEM0_SET

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\MCMEM0_ASST

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\MCMEM0_HOLD

For more information about the timings read the “PXA27x Developer Manual” section 6.5.5.

NOTE:

By default the IO timings are the slowest. There is the side effect of slow IO timings - LCD flicking. It will disappear with faster timings.

# Image File

A standard image file - NK.BIN - is generated by the platform builder. However, when a Flash Disk is available in the system (as is the case with the CM-X270), NK.BIN defines only the file system within its own range. In other words, a system based only on NK.BIN is not capable of using the rest of the Flash Disk as a filesystem. To overcome this limitation, the NK.BIN file is merged with a larger image structure that includes the surrounding filesystem. The “disk_bin_generator” program performs the merge operation. The program is provided in this archive. Please read the “disk_bin_generator” program documentation for details.

After the merge, Win CE is capable of accessing both its own components (packed in NK.BIN) and additional filesystem space free for application use.

The un-partitioned space is allocated for two images to support the Failsafe update option (read the “ImageUpdater manual.doc” in the image archive under the “Docs” directory for more details).

But only one image is active.

To create a DISK.BIN image:

- Put nk.bin and disk_bin_generator.exe in the same directory.

- Run disk_bin_generator.exe and follow the instructions in the program documentation.

The disk.bin image will be uploaded to the CM-X270 and written to NAND, as described in the Installation chapter.

# Flash Driver

#### NAND Flash Driver Notes

The following features have been added:

- Faster initialization/format time

- Support for all NAND flashes with physical block size of 2K

- Improved stability of the NAND driver

The following features have been added:

- The Nand Flash Driver cache size can be configured by setting the "NandCacheSize" value under the [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\NandFlashV3] key.

The default size is 128 NAND pages (~2Kbyte per page).

- NAND Flash Driver flush thread activity can be specified by setting the “NandFlushTimeout” value under the [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\NandFlashV3] key.

The default value is 30000 (30sec).

Normally the NAND flush is performed 200ms after the last write.

- The Nand Flash Driver flush on Suspend event can be configured by setting the

"NandFlushOnSleep“value to 1 (the recommended value).

Notes:

- The NAND Flash Driver flush is invoked automatically on the system power state transaction to D4 and D3 (see NandFlushOnSleep).

- The NAND Flash Driver flush can also be invoked from an application using global flush events:

hNandFlushCompleteEvent= OpenEvent(EVENT_ALL_ACCESS,FALSE,TEXT("NandFlushCompleteEvent"));

hNandWriteEvent= OpenEvent(EVENT_ALL_ACCESS,FALSE,TEXT("NandWriteEvent"));

if (hNandWriteEvent && hNandFlushCompleteEvent){

ResetEvent(hNandFlushCompleteEvent);

SetEvent(hNandWriteEvent);

//Sleep(0);

dwState = WaitForSingleObject (hNandFlushCompleteEvent, 5000);

if (dwState == WAIT_TIMEOUT) {

RETAILMSG(1, (TEXT("Flsuh timeout.\r\n")));

}

else {

if (dwState != WAIT_OBJECT_0) {

RETAILMSG(1, (TEXT("Wait failed (code=%d).\r\n"),GetLastError()));

}

}

}

##### NOR Flash Driver Notes

The following features have been added:

- Low Level format of the NOR will be performed during NOR driver initialization if the “LowFormat” value of  the [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\NorFlashV3] key is 1.

(The “Low Format” value will be automatically cleaned after format).

- NOR Disk size will be changed according to the “StartOffset” value of the [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\NorFlashV3] registry key. The offset is specified in NOR blocks (each NOR block is 64Kb).  Offset 0 allows allocation of the NOR Disk from block 5 in the NOR Flash (leaving 5 blocks for the boot loader).

- Initial “StartOffset” is set to 36 in order to reserve space for a splash screen.

Preserving the Registry

The provided run-time image uses a hive-based registry to store changes performed by the user or by actions such as new driver installation.

The Registry flush policy can heavily impact system performance and NAND Flash driver stability. By default, the system is configured with a Lazy Flush policy, thus an actual flush occurs every 30 seconds. You can control flush period with following registry key: [HKEY_LOCAL_MACHINE\System\ObjectStore\RegFlush]  "FlushPeriod"

If you want to perform controllable flush, you can use the RegFlush system function.

The user is advised to use the “Software Reboot” shortcut (flushes the registry before reboot), instead of performing a hardware reset, after updating the registry.

#### Asynchronous Power Off Events and NAND block corruption

The NAND Flash can be corrupted by repeated asynchronous power down events during the NAND write transaction. There are some software and hardware recommendation that can eliminate NAND corruption:

Software Solutions:

- Use RAM-based registry to eliminate asynchronous registry transactions to NAND.

- Use Lazy Flush registry policy instead of aggressive flush (flush each registry change).

You can disable the aggressive flush by writing the following registry key:

[HKEY_LOCAL_MACHINE\init\BootVars]  with  “RegistryFlags"=dword:0.

The probability of registry corruption is greatly reduced with Lazy Flush policy and asynchronous power off.

- Use software reboot (automatically flushes the NAND Flash cache):

SetSystemPowerState(“reboot”,0, 0);

- On an application level, divide store operations of large files to blocks of 128 KB each, and allow idle time between consecutive blocks (the application can proceed with other task). Such policy will write one NAND Flash block at a time and thus will minimize the flush impact of the system and decrease the probability to corrupt the other data from the file system.

Hardware Solution:

- Reset the board via GPIO instead of reset button to signal the processor to start the shutdown of NAND Flash Driver.

- Early detection of power down:

Connect a 2F (Farad) supercap on the power supply entry to keep the processor alive for a short period after power failure. Provide power fail indication through a general purpose input. When the application updates a file, it should act as follows:

  - Check power fail indication. Don't proceed if true.

  - Open the file for writing

  - Seek the file's end if necessary

  - Write to the file, limiting data length to less than 128KB

  - Close the file

Supercap will hold the power required for performing steps 2 to 5.

While calculating the required capacitance, you should assume that nominal 3.3V supply voltage is allowed to drop down to 3.1V, but not less. A 128KB (max) file write operation should be completed before the voltage drops below the critical threshold.  A 128KB write takes less than 0.3 sec. For example, assuming 1A current consumption, 0.3 sec until file closing and an additional 0.2 sec margin, the capacitor should be: (1A * 0.5 sec) / 0.2V = 2.5 farad. Supercaps of such a capacity are coin-sized.

# Embedded RegEdit

The image includes an embedded registry editor able to write/read/delete registry values.

Syntax:

Reg.exe  <(-)Registry Key> <Value> ("-" before  Registry Key will remove the value).

Where some special aliases are defined for the display key:

HKLM\Drivers\Display\PXA27x\Config

LCCR0,LCCR1,LCCR2,LCCR3,LCCR4,

CxScreen,CyScreen,Bpp,Rotation.

Where some special aliases are defined for the NOR driver key:

HKLM\Drivers\BuiltIn\NorFlashV3

StartOffset, LowFormat.

Where some special aliases are defined for the NAND Flash driver key:

HKLM\Drivers\BuiltIn\NandFlashV3

NandFlushTimeout, NandCacheSize,NandFlushOnSleep.

Where some special aliases are defined for the NAND Flash driver key:

HKLM\Drivers\BuiltIn\NandFlashV3

NandFlushTimeout, NandCacheSize,NandFlushOnSleep.

Where are some special aliases defined for Debug Enable Key:

HKLM\\Drivers\\Debug\

EnableCOMADebug (Enables debug messages on COM-A), BuildDate (Image Build Date)

Where are some special aliases defined for PCCARD SOCKET 0 Key:

HKLM\Drivers\BuiltIn\PCC_CM_X270_BSP0

MCIO0_SET,MCIO0_ASST,MCIO0_HOLD.

NOTE: The sources of reg.exe are located in CM_X270_BSP\DemoApps\RegEdit.

# Power Management

Idle Mode

When it has nothing to do, the CPU enters an "Idle" state, reducing its current consumption by 250 mA. The CPU enters and exits idle mode automatically.

During idle, the activity (red) led is off.

Suspend / Resume Mode

The platform supports the following Power Management related operations: suspend, resume and cold boot.

- Suspend reduces the current consumption to 18 mA, depending on the card’s configuration. The system goes to suspend almost instantaneously. The whole system state is saved to the SDRAM, the peripherals are either put into idle mode or disabled. On exit, the peripherals will undergo initialization and state restoration. If a certain peripheral does not operate correctly, performing the suspend/resume sequence may reset it.  An example of entering suspend mode is available in the \Suspend2\Suspend2Dlg.cpp file in the run-time image archive.

- Resume takes approximately three seconds to complete for the provided image; depending on which external devices are connected and how long it takes to initialize each one. 
Resume is initiated either upon expiration of an on-board counter, or by a falling edge signal to GPIO1.

NOTE: Do not perform repeated Resume / Suspend actions within an interval of less than 10 sec.

For more information about Suspend / Resume, please refer to the "Power Management" section of the CM-X270 Reference Guide.

There is a small memory leak during repeated Suspend/Resume operations  (~10 K per suspend/resume operation). The USB stack driver causes this memory leak during the allocation and freeing of internal queues.

Suspend / Resume can affect the functionality of USB peripherals. For more details, see the USB Support section.

# Software Reboot

Software reboot functionality (using a WATCHDOG timer) was added to the system.

There is a “Software Reboot” shortcut on the Desktop that calls the SetPowerState utility with the “reboot” parameter.

The “SetPowerState reboot” performs:

- Flushing the Registry hives

- Starting NAND Flash Driver flush

- Reboot upon a NAND flush finish event.

The sources of SetPowerState are located in CM_X270_BSP\DemoApps\SetPowerState.

# Serial Port

There are several issues influencing serial port enumeration in Win CE:

- The CM-X270W and CM-X270L have different port mappings.

- Ports can be individually enabled / disabled in registry, and only enabled ports are enumerated, in sequential order.

- One of ports (COM-B) is provided by optional PCI Bridge chip. Presence / absence of PCI Bridge option influencing port enumeration.

Ports mapping for CM-X270W:

| X270_CE5 Project Serial Port Selection | Ports assignment, including PCI Bridge | Ports assignment, excluding PCI Bridge | Support of ActiveSync Connection |
| --- | --- | --- | --- |
| PXA270 FFUART - COM-A | COM1 | COM1 | + |
| PXA270 BTUART - COM-C | COM3 | COM2 | - |
| PXA270 STUART - COM-D | COM4 | COM3 | - |
| PCI Bridge Serial - COM-B | COM2 | - | - |

Ports mapping for CM-X270L:

| X270_CE5 Project Serial Port Selection | Ports assignment, including PCI Bridge | Ports assignment, excluding PCI Bridge | Support of ActiveSync Connection |
| --- | --- | --- | --- |
| PXA270 BTUART - COM-A | COM1 | COM1 | - |
| PXA270 FFUART - COM-C | COM3 | COM2 | + |
| PXA270 STUART - COM-D | COM4 | COM3 | - |
| PCI Bridge Serial - COM-B | COM2 | - | - |

Notes:

- If only one port enabled, it will always be assigned as COM1.

- The default registry settings are for CM-X270W. In order to suite the tables above, the port mappings for CM-X270L are exchanged  (from FFUART to BTUART) automatically on boot, by the Regmod.dll driver.

- The WinCE port order is defined by driver enumeration order, that is: COM-A, COM-B, COM-C, COM-D. If some port is not selected, the WinCE enumeration order will be shifted up.

- If debug via COM-A is selected, the COM-A serial driver will be disabled automatically, and the port order will be shifted up (COM1 refer to COM-B, COM2 refer to COM-C, ..).

# Redirecting the Command Prompt Window

By default, the command prompt window is displayed on the LCD / CRT screen. It can be redirected to the COM1 port by changing the "platform.reg" file from "OutputTo"=dword:0 to "OutputTo"=dword:1. The result will take effect at the next boot.

# Analyzing the WinCE image problems

By writing “1” to HKLM\\Drivers\\Debug\EnableCOMADebug (see reg.exe utility section) the user can enable the diagnostic debug messages on COM-A (write “0” to disable).

In order to speedup the CompuLab's support, you are advised to pass the following information with each support request:

- ARMmon build date (displayed on COM-A during bootload).

- WinCE image build date (can be obtained by running “reg.exe builddate”).

- Diagnostic debug log printed on COM-A.

# WinCE Demo Image Customization

The ARMmon (releases after 01-May-2007) supports command line parameters passing to Win CE, allowing the user to perform certain image customization without image re-generation and manual registry updates.

The ARMmon syntax:

bootos “par1 par2 par3…” (parameters are not case sensitive)

The following parameters are supported currently:

| DEBUG | activate debug output on COM-A |
| --- | --- |
| BOOTIP | forces the IP address/mask setting |
| SETLCCR | forces the LCCR’s values of display driver |
| DEVMINI | image for development, includes only “must have” drivers |
| NO(driver alias) | disables the driver by its alias name |
| ADD(driver alias) | enables the driver by its alias name |

The self-explanatory driver alias names are:

PCCARD, SDCARD, SERIALA ,SERIALB, SERIALC, SERIALD, TOUCH, GPIO, I2C, USBFN, WLAN, OHCIITE, OHCIPXA, RTL8139, DM9000, NOR, NAND, AUDIO, IDE, CAMERA, LCDPXA, PCI.

# WinCE ImageUpdater Tool

The ImageUpdater utility allowing the update of the WinCE image stored in the NAND Flash. This utility is integrated in the run-time image. The user can update the NK.bin or disk.bin from a remote network drive or any attached storage device such as SD, PCMCIA or USB. Use the desktop shortcut “ImageUpdater” to run the utility.

The detailed manual  (ImageUpdater manual.doc) is found in the image archive under the “Docs” directory.

# Programming GPIO’s

GPIO Driver Access

The WinCE demo image includes GPIO driver that allows direct access to GPIO functionality. To get familiar with GPIO’s hardware you should read the “CM-X270 Reference Guide” section 5.1 “General purpose I/O”.

The GPIO driver is a stream driver that can be accessed via “GIO” prefix.

To hide the implementation details the following interface is defined (in “gpio.h”) for GPIO’s access:

- HANDLE GPIOOpen()

Creates the GPIO device handle that is used in all other functions.

- VOID GPIOClose(HANDLE hDevice)

Deletes the GPIO device handle.

- BOOL GPIOIsAvailable(HANDLE hDevice, DWORD num)

A must call function before accessing any GPIO. This function checks whether the current GPIO is free to use as GPIO and is not used by other driver (for exmple keypad driver).

- BOOL GPIOSetAsOutput(HANDLE hDevice, DWORD num, GPIO_VALUE val)

Configures the specified GPIO number(num) to be an output and sets the default value (val).

- BOOL GPIOSetAsInput(HANDLE hDevice, DWORD num)

Configures the specified GPIO number(num) to be an input.

- BOOL GPIOGetValue(HANDLE hDevice, DWORD num, GPIO_VALUE *gpioVal)

Gets the specified GPIO number(num) state. The value can be GPIO_LOW(0) or GPIO_HIGH(1).

- BOOL GPIOSetValue(HANDLE hDevice, DWORD num, GPIO_VALUE val)

Sets the specified GPIO number(num) state. The value can be GPIO_LOW(0) or GPIO_HIGH(1).

- BOOL GPIORegisterInterrupt(HANDLE hDevice, GPIO_INTERRUPT interrupt)

Registers the specified GPIO number(num) to be a source of the interrupt that will be triggered at the specified edge (see the self explanatory GPIO_EDGE_SETTINGS).  As a result the handle for interrupt event for this GPIO is returned in “GPIO_INTERRUPT.Event” member.

- BOOL GPIOUnRegisterInterrupt(HANDLE hDevice, DWORD num)

Unregisters the specified GPIO number(num) to be a source of interrupt.

- void GPIOInterruptDone(HANDLE hDevice, DWORD num)

Should be invoked for the specified GPIO number(num) at the end of each interrupt processing.

GPIO Sample Application

This GPIOSample application illustrates the most basic utilization of CM-X270 GPIO’s in Windows CE:

- How to configure GPIO pin on CM-X270 module as output and manipulate it.

- How to configure GPIO pin on CM-X270 as input and respond to changes in signal level.

- Basic polling technique.

- How to setup an event that will be triggered when the system detects an edge on an input pin.

The GPIOSample manipulate the following GPIO’s:

- GPIO 86 (pin 7 on SB-X270 P16) 	- will be set as output

- GPIO 107 (pin 45 on SB-X270 P16) 	- will be set as input

- GPIO 104 (pin 6 on SB-X270 P16) 	- will be set as input.

To test the results of GPIOSample make sure that you can check the state of GPIO86 and can control the state of GPIO107 and GPIO104.

The GPIOSample performs the following action with above GPIO’s:

- Configures an event (with GPIORegisterInterrupt) that will be signaled when any edge is detected on GPIO107 (it is also possible to detect only rising or falling edge).

- On the next step the program waits for the any edge on GPIO107. During the wait GPIO86's state is changed every 3 seconds.

- At this point, you will have to trigger an edge on GPIO107 to continue. Notice how the program signals the interrupt is done (GPIOInterruptDone) and later clears the interrupt (with GPIOUnRegisterInterrupt).

- The next phase demonstrates polling. The program will continually check the value on GPIO 104, until it will be set as "1" (externally by you), then the program will terminate.

Available GPIOs

Although the PXA270 has many GPIO’s, not all of them are available due to sharing with other functions. The GPIO driver was tested on a SB-X270 with CM-X270L and CM-X270W modules. The following tables summarize the available GPIO’s for each combination.

### Available GPIO’s on SB-X270 with CM-X270W:

| SB-X270 (P16) | PXA270 GPIO Number |
| --- | --- |
| 26 | 19 |
| 41 | 23 |
| 43 | 24 |
| 42 | 25 |
| 7 | 86 |
| 16 | 87 |
| 28 | 102 |
| 12 | 103 |
| 12 | 104 |
| 8 | 106 |
| 45 | 107 |

### Available GPIO’s on SB-X270 with CM-X270L:

| SB-X270 (P16) | PXA270 GPIO Number |
| --- | --- |
| 41 | 23 |
| 43 | 24 |
| 42 | 25 |
| 7 | 86 |
| 6 | 104 |
| 8 | 106 |
| 45 | 107 |

# Sysconf Utility

This is a command line utility, which can directly access the PXA27x registers.

In addition, the utility allows the user to load/unload drivers. It also allows the user to disable/enable certain network interfaces.

It is very useful as a debug tool, allowing the user application access to the hardware.

# Keypad Driver

Due to the lack of free GPIO’s (used for other features), the PXA270 keypad controller can’t be used. Instead the keypad driver is implemented on top of the GPIO driver and is fully customizable for any GPIO set.

Modifying the [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\Keypad] registry key can customize the keypad driver provided with the BSP. Here is a short explanation of the most important values:

| Rows | The number of rows in the matrix keypad |
| --- | --- |
| Columns | The number of columns in the matrix keypad. |
| Col[n]W | The GPIO used by column n on CM-X270W. |
| Col[n]L | The GPIO used by column n on CM-X270L |
| Row[n]W | The GPIO used by row n on CM-X270W. |
| Row[n]L | The GPIO used by row n on CM-X270L |
| Key[n] | The virtual key assigned to Key number n (the character that will be printed on the screen when key[n] is pressed) |
| DebounceTime | The delay (in mSec) set by the driver after each edge (used to avoid bounce issues) |
| Enable | When “1”, the keypad driver is enabled (the keypad is disabled by default). |

To learn more about the virtual keys please refer to -

http://www.kbdedit.com/manual/low_level_vk_list.html

The default values are set for a 3x4 matrix with all 10 digits and the characters 'a' and 'b' as its virtual keys, which are summarized in the following registry entry:

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\Keypad]

"EnableKeypad"=dword:0	;Disabled by default

"DebounceTime"=dword:14;20 mSec

"Rows"=dword:4

"Columns"=dword:3

;GPIOs for CM-X270L

"Col1L"=dword:17 ;gpio23

"Col2L"=dword:18 ;gpio24

"Col3L"=dword:19 ;gpio25

"Row1L"=dword:56 ;gpio86

"Row2L"=dword:68 ;gpio104

"Row3L"=dword:6a ;gpio106

"Row4L"=dword:6b ;gpio107

;GPIOs for CM-X270W

"Col1W"=dword:66 ;gpio102

"Col2W"=dword:67 ;gpio103

"Col3W"=dword:68 ;gpio104

"Row1W"=dword:56 ;gpio86

"Row2W"=dword:57 ;gpio87

"Row3W"=dword:6a ;gpio106

"Row4W"=dword:6b ;gpio107

"Key0"=dword:31 ;VK 1

"Key1"=dword:32 ;VK 2

"Key2"=dword:33 ;VK 3

"Key3"=dword:34 ;VK 4

"Key4"=dword:35 ;VK 5

"Key5"=dword:36 ;VK 6

"Key6"=dword:37 ;VK 7

"Key7"=dword:38 ;VK 8

"Key8"=dword:39 ;VK 9

"Key9"=dword:41 ;VK 'a'

"Key10"=dword:30 ;VK 0

"Key11"=dword:42 ;VK 'b'

NOTE:

By default, the keypad driver is disabled, because it uses almost all the available GPIO’s.

To enable the keypad driver, update the following key:

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\Keypad] “EnableKeypad” with 1.

# Notes and Errata

- To use embedded Visual C++ 4.0, you will need an appropriate SDK, which is available from CompuLab's website. A simple program may be compiled with the standard SDK as well. It will work, but will report version incompatible messages.