Custom Panel Configuration on CM-X270 with the 2700G

# Introduction to the 2700G Display Sync Controller

The display controller of 2700G Multimedia Accelerator is fully programmable.

Its sync controller drives x_LCLK (HSYNC), x_FCLK (VSYNC), and x_DEN (Data Enable).

The timing and polarity of these signals are programmable.

As with most displays, the display controller of 2700G drives pixels in

horizontal lines, starting in the upper-left corner of the display. The x_LCLK signal indicates the

beginning of a new line, and the x_FCLK signal indicates the beginning of a new frame (reset to

the upper-left corner).

There are three types of horizontal lines:

• Vertical Sync / Blanking Line

• Vertical Border Line

• Active Pixel Data Line

Each horizontal line may contain horizontal blanking, horizontal porch, horizontal border, and/or

active display data types. Horizontal lines, other than vertical blanking lines, will contain more

than one data type.

Figure 1 presents a diagram of display sync and data generation.

Figure 1.

In Figure 2, an Active Pixel Data Horizontal Line is drawn:

Figure 2.

# Registry setting for 2700G customization

The CompuLab WinCE run-time image provides registry support for a custom LCD panel.

The user should choose the “User defined” panel value in the following registry key:

[HKEY_LOCAL_MACHINE\Drivers\Display\PowerVR]

"Panel"=dword:00000009; 9 – user defined panel

The resolution and color depth should be specified here:

[HKEY_LOCAL_MACHINE\Drivers\Display\PowerVR]

"Width"=dword:280 ;400;

"Height"=dword:1E0;300;

"BitsPerPixel"=dword:10

"Panel"=dword:00000009; 9 – user defined panel

The LCD custom settings can be specified in the following key:

[HKEY_LOCAL_MACHINE\Drivers\Display\PowerVR\CustomPanelSettings]

LCD panel sync generation can be specified according to Figure 1. :

"HSync"=dword:00000060		;Horizontal sync

"HBP"=dword:00000030		;Horizontal back porch

"HLB"=dword:00000000			;Horizontal left border (typical = 0)

"HRB"=dword:00000000		;Horizontal right border (typical = 0)

"HFP"=dword:00000010			;Horizontal front porch

"HActive"=dword:00000280		;Number of active pixels in line

"VSync"=dword:00000002		;Vertical sync

"VBP"=dword:00000021			;Vertical back porch

"VTB"=dword:00000000			;Vertical top border (typical = 0)

"VBB"=dword:00000000			;Vertical bottom border (typical = 0)

"VFP"=dword:0000000a			;Vertical front porch

"VActive"=dword:000001e0		;Number of active lines in frame

The polarity of each sync signal (x_LCLK (HSYNC), x_FCLK (VSYNC), and x_DEN (Data Enable)) can be specified. The polarity value “1” means that the signal is active high and “0” means that the signal is active low.

"HSyncPol"=dword:00000001	;Polarity of Hsync (1 = positive)

"VSyncPol"=dword:00000001	;Polarity of Vsync (1 = positive)

"BlankingPol"=dword:00000001	;Polarity of Blank signal (1 = positive during blank)

From Figure 2, one can conclude that:

“HSyncPol"=dword:00000001

"BlankingPol"=dword:00000000

The LCD pixel clock (x_PCLK) should be specified in Hz. This is usually a typical pixel clock value from the data sheet:

"ClockHz"=dword:018028a8	;Dot clock in Hz

"ClockPol"=dword:00000001	;Pixel clock polarity

A ClockPol equal to “1” means that the data is issued on the rising edge of the clock and “0” means that the data is issued on the falling edge of the clock. Choose this value according to the LCD data sheet.

Double Pixel mode or Double Line mode can be selected with value “1”:

"PixelDouble"=dword:00000001	;Pixel double mode (1 = Yes, 0 = No)

"LineDouble"=dword:00000001	;Line double mode (1 = Yes, 0 = No)

To allow control of the LCD backlight independent of the state of the application processor, the

2700G provides and independently programmable pulse width modulator (PWM). The PWM period is specified in ns and should be between (77ns and 5 ms).

"BacklightPeriod"=dword:00002710	;The period of backlight PWM clock in ns

Example of settings for LTM04C380S (panel = 5) - 640x480 16bpp active matrix:

[HKEY_LOCAL_MACHINE\Drivers\Display\PowerVR\CustomPanelSettings]

"HSync"=dword:00000060		;Horizontal sync

"HBP"=dword:00000030		;Horizontal back porch

"HLB"=dword:00000000			;Horizontal left border (typical = 0)

"HRB"=dword:00000000		;Horizontal right border (typical = 0)

"HFP"=dword:00000010			;Horizontal front porch

"HActive"=dword:00000280		;Number of active pixels in line

"VSync"=dword:00000002		;Vertical sync

"VBP"=dword:00000021			;Vertical back porch

"VTB"=dword:00000000			;Vertical top border (typical = 0)

"VBB"=dword:00000000			;Vertical bottom border (typical = 0)

"VFP"=dword:0000000a			;Vertical front porch

"VActive"=dword:000001e0		;Number of active lines in frame

"ClockHz"=dword:018028a8		;Dot clock in Hz

"HSyncPol"=dword:00000001		;Polarity of Hsync (1 = positive)

"VSyncPol"=dword:00000001		;Polarity of Vsync (1 = positive)

"BlankingPol"=dword:00000001		;Polarity of Blank signal (1 = positive during blank)

"ClockPol"=dword:00000001		;Pixel clock polarity

"PixelDouble"=dword:00000000		;Pixel double mode (1 = Yes)

"LineDouble"=dword:00000000		;Line double mode (1 = Yes)

"BacklightPeriod"=dword:00002710	;The period of backlight PWM clock in ns