# Windows 5.0 on CM-X270 - GPIO example

19-July-2007

## Preliminary reading

Before trying to manipulate GPIO’s on the CM-X270 you should read “Intel® PXA270 Processor Developer’s Manual” section 24 “general purpose I/O”. This is necessary in order to understand how the GPIO registers are related to GPIOs.

## Example

This example illustrates the most basic utilization of CM-X270 GPIO’s in Windows CE.

It consists of two eMbedded Visual C++ projects:

### GPIO_output

This example demonstrates

- How to configure GPIO pin on CM-X270 module as output and manipulate it.

- How to configure GPIO pin on CM-X270 as input and respond to changes in signal level.

- Basic polling technique.

- How to setup an event that will be triggered when the system detects an edge on an input pin.

Before you start

This example will set GPIO 86 (pin 7 on SB-X270 P16) as output. GPIO 107 (pin 45 on SB-X270 P16) and GPIO 104 (pin 6 on SB-X270 P16) will be set as input. Make sure you can read the signal driven from GPIO86 and that you are able to control the signal driven into GPIO 107 and 104.

This program sets up an event signaled when any edge is detected on GPIO107 (it is also possible to detect only rising or falling edge). After that the program waits for the edge, during the wait GPIO86's value is inverted every 3 seconds, you will be able to see changes if you monitor that pin value with an oscilloscope. At this point, you will have to trigger an edge on GPIO107 to continue. Notice how the program signals the interrupt is done and later clears the interrupt.

The next phase demonstrates polling, the program will continually check the value on GPIO 104, when it will be set as logical 1 (externally by you) the program is over.