Win CE 6.0 for CM-X270

Installation & Feature Guide

This version of Windows CE is intended for evaluation purposes only. It may be used for up to 30 days. After this period, for continued use, the user must buy an appropriate Windows CE license.

# Release Notes

| 12-Jul-2007 | - First Release of Win CE 6.0 for CM-X270 - Feature functionality is equal to that of the corresponding Win CE 5.0 release, except for the 2700G. |
| --- | --- |
| 08-Aug-2007 | - Fixed Disk_bin_genarator.exe to support exFAT partition. (without exFAT support the WinCE 6.0 can’t boot from NAND) - Fixed keypad driver sleep/resume problem. - Added Display Rotation control panel applet. |
| 17-Sep-2007 | - Added new I2C bus driver that supports multiple chip access. The detail explanation can be found in demo application under Demo Applications\I2Csample\readme.txt |
| 29-Oct-2007 | - Fixed NAND Flash driver corruption problem, caused by new NAND Flash chips. The problem was analyzed and fixed for ST NAND Flash chips. |
| 26-Dec-2007 | - Fixed the GPIO driver problem. The fix includes changes in the GPIO driver and the GPIOSample application. - Fixed the I2C driver Suspend/Resume issue. The fix includes changes in the I2C driver and the I2CSample application. |
| 08-Apr-2008 | - Applications\SPI_Thermometer_ce6 - Demo Distribution was rebuilt using WinCE 6.0 R2 - New SDK added to this release - Extended the I2C driver to implement transactions of varied lengths - WiFi is now turned off at startup - Added a Com applet to enable WiFi and GPRS/GSM - Added Bluetooth support for the CM-X270W2 - Added GPRS/GSM support for the CM-X270W2 - Added the CLTerm application to test COM ports - Added an SPI driver. The SPI demo is located in Demo folder - Fixed the GPIO problem: did not release interrupt properly - Fixed System bug: random system stalls causing improper entering into CPU idle mode - Fixed AutoLaunch bug: could not launch with command line arguments |
| 26-May-2008 | - Fixed rare 22min system freezes - Fixed GPIO86 re-initialization by LCD driver - Removed Philips BGW200 Wi-Fi support due to transition to new WiFi chip (separate run-time image with that driver is still available) - Reduced CM-X270W2 support (being gradually discontinued) - Removed GPRS support - Removed Communication applet - Added support for Marvel GSPI8686 and SDIO8686 compatible onboard Wi-Fi modules. - Added new DMA subsystems allowing simultaneous execution of DMA capable devices: AUDIO, SPI Wi-Fi, SDHC - Automatic interrupt enable during NAND Flash transactions for CM-X270L Rev 1.4 (and greater) and CM-X270W Rev 1.3 (and greater). (Previously interrupts were disabled to prevent hardware integration problem with new NAND chips) - Changes in NAND Flash flush policy (see NAND section for details) - Ready to pass Windows CE 6.0 CETK BSP certification. |
| 23-Nov-2008 | - Added support for Bluetooth controller for the CM-X270W revision 1.3 platform - Added control panel applet to enable/disable the Bluetooth controller - Disabled the SPI driver for the CM-X270W revision 1.3 in favor of the WiFi module - Fixed the Xllp_I2C lengthy time outs delays. The fix can be enabled/disabled through the I2C driver’s registry settings. |

# Important Notice

Before sending a request to CompuLab's Technical Support, it is important to read the “WinCE Demo Image Troubleshoot” section at the end of this document.

# Features

## Supported Features

- NAND and NOR Flash Disks

- LCD Panels & CRT Displays

- Power Management

- PXA270 graphics using 16 bit color

- Network, with Static IP address or DHCP

- USB Slave

- USB Host ports of PXA270 and ITE8251

- Audio

- Touch Screen

- Serial Ports (COM1 – COM4)

- USB Keyboard and Mouse

- PCMCIA

- Hard disk

- RTC

- Wi-Fi driver (Marvel SPI8686 and SDIO8686 chipsets)

- GPIO stream driver

- Keypad stream driver

- I2C stream driver

- PXAReg stream driver (allows direct access to PXA27x registers)

- SPI stream driver

- Bluetooth

A run-time component list is found here.

## Features Not Supported

- CardBus (32-bit).

- 2700G is not supported in Win CE 6.0

- VIP / Quick capture interface.

# Installing the Win CE Image on the CM-X270

The following preparatory steps should to be taken:

- Connect the CRT or Flat panel monitor, USB keyboard and USB mouse to the SB-X270 baseboard.

- Connect the SB-X270’s COM-A (RJ-11 type connector on the front panel) to the serial port of the host computer, using the serial cable provided with the Evaluation Kit.

- Create the "disk.bin" image file using the "disk_bin_generator" program provided in this archive. For more information regarding the "disk.bin" file, see the "Image File" section below.

- Connect the CM-X270’s Ethernet port (P25 on the SB-X270's front panel) to the network to which your development workstation is connected.

- Ensure that you are using up-to-date ARMmon:

  - Download the ARMmon package available following the [Developer] >> [CM-X270] >> [ARMmon - image and documentation] links in CompuLab's website.

  - Check the ARMmon date of your card (reported on startup) against the latest revision note date specified in "Revision Notes" of the "ARMmon Manual" document.

    - If your ARMmon date matches the date of the latest revision - no update is required.

    - If your ARMmon date is earlier than the latest revision, then check your ARMmon date against the latest revision date of the Win CE package, as specified in this document ("Installation & Features"). If your ARMmon is older than the Win CE package - you need to update.

In other words, you should ensure that you are using either the latest version of ARMmon or at least a version not older than the Win CE package. The described procedure ensures that you will use up-to-date ARMmon, while still avoiding unnecessary updates.

Note that Evaluation Kits and cards from stock are not necessarily shipped with the latest version of ARMmon. Win CE installation won't run properly with old ARMmon.

There are several installation methods. The preferred methods utilize the Ethernet. The other method is to use UsbMLink utility.

- Installation using Ethernet (preferred)

This method is the fastest and most flexible.

It requires a running TFTP server on a host workstation running Windows 2000 or XP.

The recommended TFTPD32 server executable is supplied with this Win CE package, located in the “TFTP Server” directory.

There are two options of installation using Ethernet:

Option 1:

Store the image and partition table (disk.bin) in the NAND Flash and boot from it.  The boot process involves downloading from the NAND flash to the SDRAM.

- Using ARMmon, format the NAND Flash:

nand flash format

- Reboot the CM-X270.

- Copy the previously prepared disk.bin to the TFTP server repository.

- Using ARMmon, download the disk.bin image to NAND flash:

nand write disk.bin <IP Address of TFTP>

- Follow the "After Installation" section below.

Option 2:

Download the raw nk.bin image to RAM though the network and execute it. This procedure is well-suited to the experimental stage, when image contents are changed frequently. The procedure has to be repeated after every reboot.

- Copy the nk.bin file to the TFTP server repository.

- At the ARMonitor prompt, type the following command:

download wince tftp nk.bin <IP Address of TFTP server>

- When image downloading is complete, type bootos.

- Notes:

- The IP address of the TFTP server is displayed in your TFTPD32 GUI.

- If you use firewall software on your host PC, ensure that you unblock the TFTP Server port (defaults to port=69).

- Installation using USB link

- This method uses a USB slave port of the CM-X270 (see the SB-X270 user manual for USB3 port location and comments). It requires an ActiveSync installed on your Windows XP-based host PC.

- Installation process:

- Download the UsbMLink utility from [Developer]>>[CM-X270]>>[UsbMlink] in CompuLab's website. Following its documentation, install and activate the UsbMLink.

- Rename the "disk.bin" file (the filesystem image you created during the preparatory steps) to "nand.in". Using UsbMLink_GUI, run [NAND Update].  This will take about 15 minutes.

- Disconnect the USB from the target and follow the "After Installation" section below.

- After Installation

- Reset the CM-X270.

- To start Win CE, type "bootos" at the ARMmon prompt. If you prefer Win CE to start automatically, type “setboot os”, “save” and then “bootos”. (In this case, if you want to return to the ARMmon prompt, press Ctrl+C during reset.)

Remark: “bootos” performs a search for installed operating systems, and boots the first one it finds.  A detailed explanation is available in the ARMonitor documentation. The boot order is: check for Linux in RAM, check for Win CE in RAM, check for a Linux kernel in NOR and check for Win CE in NAND. If Linux is installed in NOR, reboot the core, enter ARMmon and perform “flash kernel” to delete the operating system from NOR.  Then “bootos” will succeed in finding Win CE on NAND.

See the “WinCE Demo Image Customization” section for details of the command line parameters for the “bootos” command.

# Image Configuration

The image available from CompuLab's website was built using a VS2005 Platform Builder applet based on a custom configuration. It contains most of the components provided by Microsoft.

The main components included in the image are:

- Network support

- PCMCIA support

- Internet Explorer

- File system Explorer

- Media Player

- Command Prompt

- Help Engine

- Control Panel

- USB Storage/HID

- IDE support

- WLAN support

- SD Memory support

- ActiveSync Communication Starter.

- CoreCon support (for application deploy/debug via ethernet)

- Remote Display

- Servers (telnet, ftp, file server,…)

- Useful utilities (reg.exe, SetPowerState.exe, GDIRotation.exe, PXARegs.exe)

- CETK support

- BlueTooth stack

The size of this Windows CE image is relatively large -- 26 MB -- because many components are included.  Typical Windows CE images with a specific purpose are much smaller and in most cases don't exceed 5-10 MB. They therefore require less NAND flash and boot faster.

# Dynamic Registry Configuration

The provided image utilizes a Hive Based Registry, which means that whatever changes are made to it during platform operation will be stored on NAND and retained after reboot.

To increase system performance and stability, the registry changes are buffered in RAM and are flushed using "lazy" policy, every 30 sec. In other words, if you make a change in the registry, it will be stored on the NAND only after 30 seconds (see “Preserving the Registry” section for details).

There are two ways to modify the registry: with programs (like installers) from the CE operating system itself or through the Remote Registry Editor. See also the “RegEdit Utility” section about a  WinCE command-line utility that can modify the registry via a Telnet session.

# Running the User Program from Flash on Start-up

- Copy your application program to NAND.

- Create and copy to the NAND a file named “StartUp.bat”.  This file should contain the command to launch an application program or any windows component.
For example: "\Windows\activesynclauncher.exe" /serial (or /serial3 on CM-X270L)
is a valid command to launch an ActiveSync application on serial port 1.

- The “MainStartUp.bat” file that is built into the image will start the StartUp.bat file from the NAND/NOR.

See also the “AutoLaunch Utility” section for an alternative automatic startup option.

# ActiveSync

The Windows CE image provided for the CM-X270 includes facilities necessary to establish a connection with a host computer. To do this, ActiveSync should be installed on the host computer. It may be downloaded from Microsoft's website: (http://www.microsoft.com/windowsmobile/activesync/activesync45.mspx)

Read the ActiveSync Troubleshoot Guide about network security configurations for your workstation to allow ActiveSync usage:

(http://www.microsoft.com/windowsmobile/help/activesync/default.mspx)

ActiveSync establishes a preliminary connection through a COM or USB port. However, from the moment a connection is established, ActiveSync will then use a network connection if one is available. It is important to remember that even when a network connection is used, the serial or USB cable should not be disconnected, as doing so will break the connection.

The following settings should be applied in the Connection Settings dialog box:

The simplest way to establish a connection between the target (CM-X270) and the host (ActiveSync) automatically is by plugging a USB cable into the USB slave port of the CM-X270. (The image is pre-configured for USB synchronization).

If you want to start synchronization via a COM port, you should double-click on the desktop shortcut “ActiveSync on COM1”. This will start the ActiveSyncLauncher.exe application.  Please read the description of this program below.

In order to establish a connection between the target (CM-X270) and the host (ActiveSync) manually, the following steps are required:

On Host (workstation)

- If you wish to use USB for ActiveSync, some preparations are required on the workstation:

  - Disconnect the USB cable between the CM-X270 and the workstation.

  - Install ActiveSync 4.5 (or later) on the workstation.

If you have an older version installed, uninstall it first.

On Target (CM-X270)

- A connection entry should be created in “Networking and Dial-up connections”.
To do this, open “Start >> Settings >> Network and Dial-up connections”, then double click on "Make New Connection". The name of the new connection is not important. The connection type should be "Direct Connection". Press “Next” and select the device to be used. Typically the device will be "Serial Cable on COM1:” which represents COM1, or “USB: ”. Press “Finish” to finalize the connection’s creation. Close the “Network and Dial-up connections” window.

- This connection should be defined as the default connection for synchronization.
To do so, enter the “Control Panel” and double click on the "PC Connection" icon. Press the “Change” button and select the connection name that was defined in step 2. Close the “Control Panel” window. (This will update the registry settings.)

- When USB is chosen as the connection transport, plug a USB cable between the CM-X270 and the host workstation. The connection will be established automatically. 
During a first time connection, be patient while Win2000 installs a new USB device.

- When establishing a connection over serial, on the ActiveSync menu, click “Get Connected….”.

- While ActiveSync is trying to establish a connection, do the following (on CM-X270): Open “Start >> Settings >> Network and Dial-up connections”, then double click on "My Connection". The connection should be established.

This procedure should be followed each time an ActiveSync connection is required. In order to simplify this procedure, a utility to perform these steps was written. The utility is called ActiveSyncLauncher and can operate with COM ports only (it is not relevant for USB!).

In the command line, ActiveSyncLauncher receives the name of the device to be used to connect to ActiveSync. If nothing is specified, ActiveSyncLauncher assumes that the connection is via COM1.

For convenience, the icon is provided on the desktop. Double clicking on "ActiveSync on COM1"

launches ActiveSyncLauncher.exe.

ActiveSync requires full modem control line support from the serial port; thus, the only serial port suitable for ActiveSync is COM1.

The sources of ActiveSyncLauncher.exe are located in C:\WINCE600\OSDesigns\x270_CE6\Subprojects\ ActiveSyncLauncher and are available with a BSP package.

# Application Development with Demo Image

This package includes MFC demo applications written with VS2005, which are located in the “Demo Applications\” directory in the x270-wince6-img archive. The applications are:

- GPIOSample	- This application illustrates the use of a GPIO driver to control the state

of available GPIO’s.

- RTCResume	- This application demonstrates suspend/resume functionality using the 
 		   RTC alarm event.

Before you start with demo applications the CompuLab SDK “X270_CE6_SDK.msi” must be installed. See the “Developing WinCE 6.0 Application .doc” for the details of application develop/debug/deploy process.

# USB Support

A host USB driver is included in the Win CE image. However, the host USB driver alone is not sufficient for peripheral interfacing.  You also need a peripheral Device Driver, which should be obtained from the peripheral's manufacturer. The sample image includes all USB device drivers provided by Microsoft: keyboard, mouse, USB storage, smart card reader and USB printer driver. For any other peripheral support, please check with the peripheral’s manufacturer for device driver availability.

Some brands of USB keyboards may not initialize correctly, if they were connected to the USB during board initialization (booting of OS). Disconnecting or reconnecting the keyboard usually solves the problem.

After SUSPEND, it may take a few seconds for the USB keyboard and mouse to become operational. This delay does not interfere with the execution of background tasks.

The USB3 port of the CM-X270 can be configured as a slave (default) or as a host. Selection of host or slave mode is performed by ARMmon and should be accompanied by proper jumpers settings.

To change the USB3 configuration to host:

- In ARMmon, type:  “usb3mode host” and “save”.

- Power off the SB-X270 and shorten the pin10 with pin11 on P15 by a jumper.

To change the USB3 configuration back to slave:

- In ARMmon, type:  “usb3mode slave” and “save”.

- Power off the SB-X270 and remove the jumper (if any) from pin10, pin11 on P15.

# Ethernet

The sample Windows CE image is configured to use DHCP to get network settings for both network adapters. The network settings can be changed from the Windows CE Control Panel.

Alternately you can configure the static IP option via registry. For example:

[HKEY_LOCAL_MACHINE\Comm\DM9CE1\Parms\TcpIp]

"EnableDHCP"=dword:0

"IpAddress"="10.1.1.45"

"Subnetmask"="255.255.255.0"

"DefaultGateway"="10.1.1.1"

configures “static ip” and “subnet mask “  and  “default gateway” for DM9000 core ethernet adapter

# Wi-Fi

The WinCE release includes a driver for the Marvel GSPI8686 WiFi controller integrated into the CM-X270. The controller communicates with the CPU through an SPI interface.

The run-time image is configured to use DHCP.

The static network adapter setting can be specified in the following registry key:

[HKEY_LOCAL_MACHINE\Comm\GSPI86861\Parms\TcpIp]

"EnableDHCP"=dword:0

"IpAddress"="10.1.1.44"

"Subnetmask"="255.255.255.0"

"DefaultGateway"="10.1.1.1"

"DNS"="212.150.48.169"

The GSPI8686 configuration information can be specified in the following registry key:

[HKEY_LOCAL_MACHINE\Comm\SDIO86861\Parms\].

NOTE: Sleep/Resume is currently not supported for Marvel Wi-Fi driver.

Driver for older cards using BGW200 WiFi chip is still available in separate run-time image. It is controlled by registry keys:

[HKEY_LOCAL_MACHINE\Comm\WLAN1\Parms\TCPIP]

[HKEY_LOCAL_MACHINE\Comm\WLAN1\Parms\]

# PXA270 built-in LCD controller

Selection between the PXA270 LCD controller and the optional 2700G Multimedia Accelerator is performed by ARMmon and can be controlled by the "setgraphics" command. (The actual selection activates the regmod.dll driver that starts before GWES.)

Default display resolution is 640 x 480 and can be changed by rebuilding the image with different registry settings.  The default settings are directly compatible with VGA CRT displays and certain types of TFT LCD panels, such as Toshiba’s LTM04C380K.

An example of display registry settings (available in "display_pxa270.inc" in the BSP):

[HKEY_LOCAL_MACHINE\Drivers\Display\PXA27x\Config]

"DisplayDll"="pxa27x_lcd.dll"

"CxScreen"=dword:280; dec 640

"CyScreen"=dword:1E0; dec 480

"Bpp"=dword:10 ;16 bpp

The “CxScreen” and “CyScreen” specify the display resolution.

Bpp (Bits Per Pixel) settings

The current version of pxa270_lcd.dll supports only 16 bit colors.

Rotation

The pxa270_lcd.dll can perform GDI rotation with the following registry key:

[HKEY_LOCAL_MACHINE\System\GDI\Rotation]

"Angle"=dword:0

Possible values are: 0, 5A , B4, 10E (0,90,180 and 270 degree)

NOTE:

- Changes in the registry will take effect after reboot.

- The actual rotation will be performed by the GDIRotation.exe utility launched automatically after the GWES.

Customizing LCD timings

The PXA270 LCD timings can be customized with the following registers:

"LCCR0"=dword:

"LCCR1"=dword:

"LCCR2"=dword:

"LCCR3"=dword:

"LCCR4"=dword:

These values supersede the defaults established by the “DefaultDisplayType”.

Usually the user obtains these values from CompuLab and is then required to update the registry.

To update the LCCR’s via Ethernet connection for headless target, perform the following steps:

- Set the static ip in ARMmon for example:

“setip ip 10.1.1.45”

“setip mask 255.255.255.0”

- Boot WinCE with “bootos bootip” option (will set the static IP address for the Ethernet driver). See the “WinCE Demo Image Customization” section for details.

- Establish a Telnet session to the target (use the static IP from ARMmon).

- Update LCCR’s by typing in the telnet window:

“reg lccr0 h<value> “ (add “h” for hex number value).

See the “RegEdit Utility” section for details of reg.exe usage.

NOTE:	If you know the target IP, you may skip steps 1-2.

Another way to set up the LCCR values for the first time is to use the LCCR values from ARMmon (if you can see the valid splash screen picture in ARMmon) and force the WinCE image to update the registry with these values.

- From ARMmon, type “bootos setlccr”.

See the “WinCE Demo Image Customization” section for details.

There are many displays that are supported (the proper LCCR’s are configured according to the datasheets). A more detailed list is available following the

[Developer] >> [X270] >> [Verified Panels List] links in CompuLab's website. Support of additional panels can be obtained using CompuLab's panel adaptation service.

# 2700G Multimedia Accelerator

The 2700G Multimedia Accelerator is currently not supported in Win CE 6.0.

# Splash Screen

Splash screen is a term used to describe an image that appears while the OS is loading, so the display would not stay black. To enable splash screen display, the user should setup ARMmon to copy the splash screen image from the NOR to the RAM. For details see the "Splash Screen" section in the ARMmon manual.

To avoid bootloader from overriding splash screen RAM during o/s boot, 1MB of RAM is reserved starting from address 0xA3D00000. During evaluation, the splash screen image should be loaded  at 0xA3D00000 by the following ARMmon command: “splashscreen dram A3D00000”

Explanation: a common problem is maintaining the splash screen image on the screen during WinCE boot. During boot, the WinCE image (NK.bin) is loaded into RAM and then invalidates all available RAM (according to config.bib) at the early boot phase. If the splash screen is loaded (by ARMmon) into the RAM region accessed by the WinCE image, it will be removed or corrupted.

NOTE:

If you don't use the splash screen and want to free up the reserved 1MB of RAM, you can turn off this feature with the BSP_NOSPLASH environment variable in BSP (See the “Customizing the WinCE Image” section of BSP.doc provided with BSP).

# Touch screen

A driver for resistive touch screens is available in the provided package. The CM-X270 must contain the "AT" option to provide touch screen support.

Note: Some touch screens are very sensitive to electro-magnetic interference. The driver filters out sequences of samples that have large variance. Therefore, in a noisy environment, the user will find the touch screen unresponsive and sluggish. For example, in a clean environment, the driver provides 40 samples per second, whereas in a noisy environment (e.g., the LCD back light inverter has been turned on), only 20 samples per second are accepted.

On the CM-X270 without AC97 codec (i.e., without the "A" or "AT" options), it is necessary to rebuild the image with the "Exclude from build and Image" option for both Audio and Touch controller drivers. If the drivers are not excluded, the CM-X270 without AC97 codec will not be able to enter sleep mode.

# Programming GPIO’s

GPIO Driver Access

The WinCE demo image includes GPIO driver that allows direct access to GPIO functionality. To get familiar with GPIO’s hardware you should read the “CM-X270 Reference Guide” section 5.1 “General purpose I/O”.

The GPIO driver is a stream driver that can be accessed via “GIO” prefix.

To hide the implementation details the following interface is defined (in “gpio.h”) for GPIO’s access:

- HANDLE GPIOOpen()

Creates the GPIO device handle that is used in all other functions.

- VOID GPIOClose(HANDLE hDevice)

Deletes the GPIO device handle.

- BOOL GPIOIsAvailable(HANDLE hDevice, DWORD num)

A must call function before accessing any GPIO. This function checks whether the current GPIO is free to use as GPIO and is not used by other driver (for exmple keypad driver).

- BOOL GPIOSetAsOutput(HANDLE hDevice, DWORD num, GPIO_VALUE val)

Configures the specified GPIO number(num) to be an output and sets the default value (val).

- BOOL GPIOSetAsInput(HANDLE hDevice, DWORD num)

Configures the specified GPIO number(num) to be an input.

- BOOL GPIOGetValue(HANDLE hDevice, DWORD num, GPIO_VALUE *gpioVal)

Gets the specified GPIO number(num) state. The value can be GPIO_LOW(0) or GPIO_HIGH(1).

- BOOL GPIOSetValue(HANDLE hDevice, DWORD num, GPIO_VALUE val)

Sets the specified GPIO number(num) state. The value can be GPIO_LOW(0) or GPIO_HIGH(1).

- BOOL GPIORegisterInterrupt(HANDLE hDevice, GPIO_INTERRUPT interrupt)

Registers the specified GPIO number(num) to be a source of the interrupt that will be triggered at the specified edge (see the self explanatory GPIO_EDGE_SETTINGS).  As a result the handle for interrupt event for this GPIO is returned in “GPIO_INTERRUPT.Event” member.

- BOOL GPIOUnRegisterInterrupt(HANDLE hDevice, DWORD num)

Unregisters the specified GPIO number(num) to be a source of interrupt.

- void GPIOInterruptDone(HANDLE hDevice, DWORD num)

Should be invoked for the specified GPIO number(num) at the end of each interrupt processing.

GPIO Sample Application

This GPIOSample application illustrates the most basic utilization of CM-X270 GPIO’s in Windows CE:

- How to configure GPIO pin on CM-X270 module as output and manipulate it.

- How to configure GPIO pin on CM-X270 as input and respond to changes in signal level.

- Basic polling technique.

- How to setup an event that will be triggered when the system detects an edge on an input pin.

The GPIOSample manipulate the following GPIO’s:

- GPIO 86 (pin 7 on SB-X270 P16) 	- will be set as output

- GPIO 107 (pin 45 on SB-X270 P16) 	- will be set as input

- GPIO 104 (pin 6 on SB-X270 P16) 	- will be set as input.

To test the results of GPIOSample make sure that you can check the state of GPIO86 and can control the state of GPIO107 and GPIO104.

The GPIOSample performs the following action with above GPIO’s:

- Configures an event (with GPIORegisterInterrupt) that will be signaled when any edge is detected on GPIO107 (it is also possible to detect only rising or falling edge).

- On the next step the program waits for the any edge on GPIO107. During the wait GPIO86's state is changed every 3 seconds.

- At this point, you will have to trigger an edge on GPIO107 to continue. Notice how the program signals the interrupt is done (GPIOInterruptDone) and later clears the interrupt (with GPIOUnRegisterInterrupt).

- The next phase demonstrates polling. The program will continually check the value on GPIO 104, until it will be set as "1" (externally by you), then the program will terminate.

Available GPIOs

Although the PXA270 has many GPIO’s, not all of them are available due to sharing with other functions. The GPIO driver was tested on a SB-X270 with CM-X270L and CM-X270W modules. The following tables summarize the available GPIO’s for each combination.

### Available GPIO’s on SB-X270 with CM-X270W:

| SB-X270 (P16) | PXA270 GPIO Number |
| --- | --- |
| 26 | 19 |
| 41 | 23 |
| 43 | 24 |
| 42 | 25 |
| 7 | 86 |
| 16 | 87 |
| 28 | 102 |
| 12 | 103 |
| 12 | 104 |
| 8 | 106 |
| 45 | 107 |

### Available GPIO’s on SB-X270 with CM-X270L:

| SB-X270 (P16) | PXA270 GPIO Number |
| --- | --- |
| 41 | 23 |
| 43 | 24 |
| 42 | 25 |
| 7 | 86 |
| 6 | 104 |
| 8 | 106 |
| 45 | 107 |

# Keypad Driver

Due to the lack of free GPIO’s (used for other features), the PXA270 keypad controller can’t be used. Instead the keypad driver is implemented on top of the GPIO driver and is fully customizable for any GPIO set.

Modifying the [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\Keypad] registry key can customize the keypad driver provided with the BSP. Here is a short explanation of the most important values:

| Rows | The number of rows in the matrix keypad |
| --- | --- |
| Columns | The number of columns in the matrix keypad. |
| Col[n]W | The GPIO used by column n on CM-X270W. |
| Col[n]L | The GPIO used by column n on CM-X270L |
| Row[n]W | The GPIO used by row n on CM-X270W. |
| Row[n]L | The GPIO used by row n on CM-X270L |
| Key[n] | The virtual key assigned to Key number n (the character that will be printed on the screen when key[n] is pressed) |
| DebounceTime | The delay (in mSec) set by the driver after each edge (used to avoid bounce issues) |
| Enable | When “1”, the keypad driver is enabled (the keypad is disabled by default). |

To learn more about the virtual keys please refer to -

http://www.kbdedit.com/manual/low_level_vk_list.html

The default values are set for a 3x4 matrix with all 10 digits and the characters 'a' and 'b' as its virtual keys, which are summarized in the following registry entry:

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\Keypad]

"EnableKeypad"=dword:0	;Disabled by default

"DebounceTime"=dword:14;20 mSec

"Rows"=dword:4

"Columns"=dword:3

;GPIOs for CM-X270L

"Col1L"=dword:17 ;gpio23

"Col2L"=dword:18 ;gpio24

"Col3L"=dword:19 ;gpio25

"Row1L"=dword:56 ;gpio86

"Row2L"=dword:68 ;gpio104

"Row3L"=dword:6a ;gpio106

"Row4L"=dword:6b ;gpio107

;GPIOs for CM-X270W

"Col1W"=dword:66 ;gpio102

"Col2W"=dword:67 ;gpio103

"Col3W"=dword:68 ;gpio104

"Row1W"=dword:56 ;gpio86

"Row2W"=dword:57 ;gpio87

"Row3W"=dword:6a ;gpio106

"Row4W"=dword:6b ;gpio107

"Key0"=dword:31 ;VK 1

"Key1"=dword:32 ;VK 2

"Key2"=dword:33 ;VK 3

"Key3"=dword:34 ;VK 4

"Key4"=dword:35 ;VK 5

"Key5"=dword:36 ;VK 6

"Key6"=dword:37 ;VK 7

"Key7"=dword:38 ;VK 8

"Key8"=dword:39 ;VK 9

"Key9"=dword:41 ;VK 'a'

"Key10"=dword:30 ;VK 0

"Key11"=dword:42 ;VK 'b'

NOTE:

By default, the keypad driver is disabled, because it uses almost all the available GPIO’s.

To enable the keypad driver, update the following key:

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\Keypad] “EnableKeypad” with 1.

SPI Driver

The WinCE demo image includes an SPI driver that allows SPI port utilization. To become familiar with SPI hardware, you should read the “CM-X270 Reference Guide” section 3.5, “SSP - SYNCHRONOUS SERIAL PORT INTERFACE”.

The SPI driver is a stream driver that can be accessed using the “SPI” prefix.

To hide the implementation details, the following interface is defined (in “spi.h”) for SPI access:

- HANDLE SPIUserOpen(TCHAR *strName)

Creates the SPI device handle that is used in all other functions. StrName should be “SPI1:”.

- void SPIUserClose(HANDLE hContext)

Deletes the SPI device handle

- BOOL SPIUserTransact(HANDLE hDevice, SPITRANS* pTrans)

This asks the driver to execute the transaction described in the struct pTrans.

A detailed explanation of this struct and its use can be found in the “SPI Demo Application” document, located in the Demo applications\SPISample folder.

SPI Sample Application

Please refer to the “SPI Demo Application” document located in Demo applications\SPISample, in order to become familiar with the demo of the SPI and use of the driver.

PCMCIA

The PCMCIA timings can have a critical impact on system stability.

Too fast timings can cause system instability and too slow timings can cause LCD flickering and slow performance of a PCMCIA device.

Fine tuning of the timings should be performed for the specific PCMCIA card used in the customer’s system. The following registry settings are used to control the PCMCIA timings:

IO Timings

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\CM_X270_PCCARD\

“MCIO0_SET”

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\CM_X270_PCCARD\

“MCIO0_ASST”

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\CM_X270_PCCARD\

“MCIO0_HOLD”

Attribute Timings

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\CM_X270_PCCARD\

“MCATT0_SET”

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\CM_X270_PCCARD\

“MCATT0_ASST”

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\CM_X270_PCCARD\

“MCATT0_HOLD”

Memory timings

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\CM_X270_PCCARD\

“MCMEM0_SET”

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\CM_X270_PCCARD\

“MCMEM0_ASST”

[HKEY_LOCAL_MACHINE\Drivers\BuiltIn\CM_X270_PCCARD\

“MCMEM0_HOLD”

For more information about the timings, read the “PXA27x Developer Manual” section 6.5.5.

# Image File

A standard image file - NK.BIN - is generated by the platform builder. However, when a Flash Disk is available in the system (as is the case with the CM-X270), NK.BIN defines only the file system within its own range. In other words, a system based only on NK.BIN is not capable of using the rest of the Flash Disk as a file system. To overcome this limitation, the NK.BIN file is merged with a larger image structure that includes the surrounding file system. The “disk_bin_generator” program performs the merge operation. The program is provided in this archive. Please read the “disk_bin_generator” program documentation for more details.

After the merge, Win CE is capable of accessing both its own components (packed in NK.BIN) and additional file system space free for application use.

The unpartitioned space is allocated for two images to support the Failsafe update option (read the “ImageUpdater manual.doc” in the image archive under the “Docs” directory for more details).

Only one image is active.

To create a DISK.BIN image:

- Put nk.bin and disk_bin_generator.exe in the same directory.

- Run disk_bin_generator.exe and follow the instructions in the program documentation.

The disk.bin image will be uploaded to the CM-X270 and written to NAND, as described in the Installation section.

# Flash Driver

NAND Flash Driver Notes

- The NAND Flash Driver cache size can be configured by setting the "NandCacheSize" value under the [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\NandFlashV3] key.

The default size is 128 NAND pages (~2Kbyte per page).

- The NAND Flash Driver flush thread activity can be specified by setting the “NandFlushTimeout” value under the following key: [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\NandFlashV3]

The default value is 200ms. Normally, the NAND flush is performed 200ms after the last write, thus allowing the file system 200ms to fill internal 300K cache in order to increase the write performance.

The internal cache flush can be estimated as 200ms (after last write) + 1-2sec of actual page flushing to the NAND  (1-2 sec = 300 Kb of cache data/ average transfer rate of 300K/s).

The NAND Flash Driver disables flush before suspending (enters D3) and enables the flush on resume (enters D0).

- The NAND Flash Driver automatically performs flush on reboot (enters D4).

Can be invoked by setpowerstate “reboot”.

##### NOR Flash Driver Notes

- Low Level format of the NOR will be performed during NOR driver initialization if the “LowFormat” value of the [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\NorFlashV3] key is 1.

(The “Low Format” value will be automatically cleaned after format).

- NOR Disk size will be changed according to the “StartOffset” value of the [HKEY_LOCAL_MACHINE\Drivers\BuiltIn\NorFlashV3] registry key. The offset is specified in NOR blocks (each NOR block is 64Kb).  Offset 0 allows allocation of the NOR Disk from block 5 in the NOR Flash (leaving 5 blocks for the boot loader).

- Initial “StartOffset” is set to 36 in order to reserve space for a splash screen.

Preserving the Registry

The provided run-time image uses a hive-based registry to store changes performed by the user or made by actions such as new driver installation.

The Registry flush policy can significantly affect system performance and NAND Flash driver stability. By default, the system is configured with a Lazy Flush policy, thus an actual flush occurs every 30 seconds. You can control the flush period with the following registry key: [HKEY_LOCAL_MACHINE\System\ObjectStore\RegFlush]  "FlushPeriod"

If you want to perform controllable flush, you can use the RegFlush system function.

The user should use the “Software Reboot” shortcut (flushes the registry before reboot), instead of performing a hardware reset, after updating the registry.

#### Asynchronous Power Off Events and NAND block corruption

The NAND Flash can be corrupted by repeated asynchronous power down events during the NAND write transaction. There are some software and hardware recommendations that can prevent such NAND corruption:

Software Solutions:

- Use a RAM-based registry to eliminate asynchronous registry transactions to NAND.

- Use a Lazy Flush registry policy instead of aggressive flush (flush each registry change).

You can disable the aggressive flush by writing the following registry key:

[HKEY_LOCAL_MACHINE\init\BootVars]  with  “RegistryFlags"=dword:0.

The probability of registry corruption is greatly reduced with a Lazy Flush policy.

- Use software reboot (automatically flushes the NAND Flash cache):

SetSystemPowerState(“reboot”,0, 0);  Avoid hardware resets.

- On an application level, divide store operations of large files to blocks of 128 KB each, and allow idle time between consecutive blocks (the application can proceed with other tasks). Such a policy will write one NAND Flash block at a time and thus will minimize the flush impact of the system and decrease the probability of corrupting other data from the file system.

Hardware Solutions:

- Restart the board via GPIO instead of a reset button, to signal the processor to start the shutdown of the NAND Flash Driver.

- Early detection of power down:

Connect a 2F (Farad) supercap on the power supply entry to keep the processor running for a short period after power failure. Provide power fail indication through a general purpose input. The sequence of writing to files should be as follows:

  - Check power fail indication. Don't proceed if true.

  - Open the file for writing

  - Seek the file's end if necessary

  - Write to the file, limiting data length to less than 128KB

  - Close the file

The supercap will hold the power required for performing steps 2 to 5.

When calculating the required capacitance, you should assume that nominal 3.3V supply voltage is allowed to drop down to 3.1V, but not less. A 128KB (max) file write operation should be completed before the voltage drops below the critical threshold.  A 128KB write takes less than 0.3 sec. For example, assuming 1A current consumption, 0.3 sec until file closing and an additional 0.2 sec margin, the capacitor should be: (1A * 0.5 sec) / 0.2V = 2.5 farad. Supercaps of such a capacity are coin-sized.

# Power Management

Idle Mode

When it has nothing to do, the CPU enters an "Idle" state, reducing its current consumption by 250 mA. The CPU enters and exits idle mode automatically.

During idle, the activity (red) led is off.

Suspend / Resume Mode

The platform supports the following Power Management related operations: suspend, resume and cold boot.

- Suspend reduces the current consumption to a minimum of 18 mA, depending on the card’s configuration. The system goes to suspend almost instantaneously. The whole system state is saved to the SDRAM, the peripherals are either put into idle mode or disabled. On exit, the peripherals will undergo initialization and state restoration. If a certain peripheral does not operate correctly, performing the suspend/resume sequence may reset it.  An example of entering suspend mode is available in the \Demo Applications\RTCResume directory in the run-time image archive.

- Resume takes approximately three seconds to complete for the provided image; depending on which external devices are connected and how long it takes to initialize each one. 
Resume is initiated either upon expiration of an on-board counter, or by a falling edge signal to GPIO1.

NOTE: Do not perform repeated Resume / Suspend actions within an interval of less than 10 sec.

For more information about Suspend / Resume, please refer to the "Power Management" section of the CM-X270 Reference Guide.

There is a small memory leak during repeated Suspend/Resume operations  (~10 K per suspend/resume operation). The USB stack driver causes this memory leak during the allocation and freeing of internal queues.

Suspend / Resume can affect the functionality of USB peripherals. For more details, see the USB Support section.

# Software Reboot

Software reboot functionality (using a WATCHDOG timer) has been added to the system.

There is a “Software Reboot” shortcut on the Desktop that calls the SetPowerState utility with the “reboot” parameter.

SetPowerState “reboot” performs the following:

- Flushes the Registry hives and

- Brings the system into power state “Reboot”.

This includes automatic NAND flush by the NAND Flash Driver.

See the “SetPowerState Utility” section for more information.

# Serial Port

There are several issues influencing serial port enumeration in Win CE:

- The CM-X270W and CM-X270L have different port mappings.

- Ports can be individually enabled / disabled in registry, and only enabled ports are enumerated, in sequential order.

- One of ports (COM-B) is provided by an optional PCI Bridge chip. The presence / absence of the PCI Bridge option influences port enumeration.

Port mapping for the CM-X270W:

| X270_CE6 Project Serial Port Selection | Port assignment, including PCI Bridge | Port assignment, excluding PCI Bridge | Support of ActiveSync Connection |
| --- | --- | --- | --- |
| PXA270 FFUART - COM-A | COM1 | COM1 | + |
| PXA270 BTUART - COM-C (GPRS) | COM3 | COM2 | - |
| PXA270 STUART - COM-D | COM4 | COM3 | - |
| PCI Bridge Serial - COM-B | COM2 | - | - |

Port mapping for the CM-X270L:

| X270_CE6 Project Serial Port Selection | Port assignment, including PCI Bridge | Port assignment, excluding PCI Bridge | Support of ActiveSync Connection |
| --- | --- | --- | --- |
| PXA270 BTUART - COM-A | COM1 | COM1 | - |
| PXA270 FFUART - COM-C | COM3 | COM2 | + |
| PXA270 STUART - COM-D | COM4 | COM3 | - |
| PCI Bridge Serial - COM-B | COM2 | - | - |

Notes:

- If only one port is enabled, it will always be assigned COM1.

- The default registry settings are for the CM-X270W. In order to suit the tables above, the port mappings for the CM-X270L are exchanged  (from FFUART to BTUART) automatically upon boot by the Regmod.dll driver.

- The WinCE port order is defined by driver enumeration order, that is: COM-A, COM-B, COM-C, COM-D. If a port is not selected, the WinCE enumeration order will be shifted up.

- If debug via COM-A is selected, the COM-A serial driver will be disabled automatically and the port order will be shifted up (COM1 refers to COM-B, COM2 refers to COM-C, etc.).

The easiest way to test serial port connection is by using the integrated terminal program CL Terminal. You can find the CL Terminal program under Start >> Programs

In the CL Terminal application, press the Config button to select the appropriate COM port and baud rate. After you open the COM port with Open button, you can feed the text at the Xmit field.

The response will be printed in the main window.

#### Bluetooth Evaluation

The BlueTooth module can be activated through the “Bluetooth Controller Enable” applet. Located in the control panel.

The Demo Distribution includes the following Microsoft Bluetooth profiles:

- Object Push Profile (OBEX)

- File Transfer Profile

- Human Interface Device Profile

- LAN Access Profile

- Personal Area Network Profile

- Hands-free and headset Profiles

You can find, pair and connect other devices using the Bluetooth Manager located in Start >> Settings >> Control Panel >> Bluetooth Device Properties.

In order to evaluate the profile’s functionality, you should write the Bluetooth Profile aware application. You can find a lot of Microsoft Bluetooth samples at WinCE 6.0 shared source code, which comes with the Platform Builder installation (C:\WINCE600\PUBLIC\COMMON\OAK\DRIVERS\BLUETOOTH\SAMPLE).

In order to evaluate Bluetooth connectivity, the following devices were used:

- Motorola V6 Cellphone

- Motorola HT820 Bluetooth Stereo Headphones

In order to test the BlueTooth Audio profile, you should use the AdpMgr Sample, which is located at Start >> Programs >> A2dpMgr.

# Redirecting the Command Prompt Window

By default, the command prompt window is displayed on the LCD / CRT screen. It can be redirected to the COM1 port by changing the following registry value:

[HKEY_LOCAL_MACHINE\Drivers\Console]

OutputTo = REG_DWORD:1 ;Redirects CMD to COM1

The result will take effect at the next boot.

# WinCE Demo Image Customization

The ARMmon (releases after 01-May-2007) supports command line parameters passing to Win CE, allowing the user to perform certain image customization without image re-generation and manual registry updates.

The ARMmon syntax:

bootos “par1 par2 par3…” (parameters are not case sensitive)

The following parameters are supported currently:

| DEBUG | Activate debug output on COM-A |
| --- | --- |
| BOOTIP | Forces the IP address/mask setting from ARMmon to WinCE |
| SETLCCR | Forces the LCCR values of the display driver |
| SAFEMODE | Includes only “must have” drivers |
| CLEAN | Cleans registry hives |
| NO(driver alias) | Disables the driver by its alias name |
| ADD(driver alias) | Enables the driver by its alias name. The self-explanatory driver alias names are: PCCARD, SDCARD, SERIALA ,SERIALB, SERIALC, SERIALD, TOUCH, GPIO, I2C, USBFN, WLAN, OHCIITE, OHCIPXA, RTL8139, DM9000, NOR, NAND, AUDIO, IDE, LCDPXA, PCI |

# WinCE Demo Image Troubleshoot

Symptom:  No WinCE GUI on LCD

Analysis 1: 	You are probably running with incorrect display driver settings: LCCR’s, CxScreen, CyScreen or Bpp.

Ensure that you have the correct settings for your LCD.

Symptom:  The Image does not boot

Analysis 1:	What is the state of  red and green LED’s on the CM-X270 module?

    - Green LED blinking ~ every 2 seconds means that the Kernel Scheduler is working properly. This is the normal state.

    - The red LED signals system activity level.

Constant red means the system is working or some driver/application is stuck (for example while (1)).

Analysis 2: 	Enable Debug Messages with “bootos debug”

Try to identify the problematic driver according to the debug output in the terminal window

Analysis 3:	If you do not see “REGMOD: Starting REGMOD Scheduler” in the log, you may have a problem with the registry hives on the NAND.

Perform the following steps:

    - Boot with "bootos clean debug" to clean the registry hive

    - If the previous step does not help, perform the NAND flash low level format with “nand flash format” command in ARMmon, and then reinstall Win CE image.

Analysis 4:	Try to boot with the minimum set of drivers

Boot with "bootos safemode debug"

Analysis 5: 	If the “bootos safemode” helps, try to find the problematic driver.

Disable the drivers one by one, for example  "bootos nowlan debug” will boot without the Wi-Fi driver.

In order to speed up CompuLab's support, you are advised to include the following information with each support request:

- ARMmon build date (reported through COM-A during bootload)

- WinCE image build date (can be obtained by running “reg.exe builddate”)

- Diagnostic debug log printed on COM-A

- Your observations following the above suggestions

# WinCE ImageUpdater Tool

The ImageUpdater utility allows the update of the WinCE image stored in the NAND Flash. This utility is integrated in the run-time image. The user can update NK.bin or disk.bin from a remote network drive or any attached storage device such as SD, PCMCIA or USB. Use the desktop shortcut “ImageUpdater” to run the utility.

A detailed manual  (ImageUpdater manual.doc) is found in the image archive under the “Docs” directory.

# RegEdit Utility

The image includes an embedded registry editor able to write/read/delete registry values.

Syntax:  reg.exe <(-)Registry Key> (h) <Value>

("-" before  Registry Key will remove the value, “h” for hex value).

There are some special aliases defined for the display key:

HKLM\Drivers\Display\PXA27x\Config

LCCR0,LCCR1,LCCR2,LCCR3,LCCR4,

CxScreen,CyScreen,Bpp,Rotation.

There are some special aliases defined for GDI display rotation angle:

HKLM\System\GDI\Rotation\

Angle.

There are some special aliases defined for the NOR driver key:

HKLM\Drivers\BuiltIn\NorFlashV3

StartOffset, LowFormat.

There are some special aliases defined for the NAND Flash driver key:

HKLM\Drivers\BuiltIn\NandFlashV3

NandFlushTimeout, NandCacheSize.

There are some special aliases defined for Debug Enable Key:

HKLM\\Drivers\\Debug\

EnableCOMADebug (Enables debug messages on COM-A), BuildDate (Image Build Date)

There are some special aliases defined for PCCARD SOCKET 0 Key:

HKLM\Drivers\BuiltIn\CM_X270_PCCARD

MCIO0_SET, MCIO0_ASST, MCIO0_HOLD.

The sources of reg.exe are located in C:\WINCE600\OSDesigns\x270_CE6\Subprojects\RegEdit\ (and are available with a BSP package).

# SetPowerState Utility

This is a command line utility, which can perform global system power state transitions.

Syntax:  SetPowerState.exe <State> <HexStateFlags> <HexOptions>

where <State> is system power state defined in the registry, e.g. Reboot.

optional <StateFlags> specify state flags, in hex, e.g.200000 ( POWER_STATE_SUSPEND).

optional <Options> specify options, in hex, e.g. 1000 (POWER_FORCE),

The most useful transitions are:

“SetPowerState reboot”	 – reboots the CM-X270 with the help of a watchdog

“SetPowerState suspend”	 – enters PXA27X deep sleep mode

Before power state transition, the registry is flushed to NAND.

The sources of SetPowerState.exe are located in C:\WINCE600\OSDesigns\x270_CE6\Subprojects\SetPowerState\

and are also available with a BSP package.

# SYSCONF Utility

This is a command line utility, which can directly access the PXA27x registers.

In addition, the utility allows the user to load/unload drivers. It also allows the user to disable/enable certain network interfaces.

It is very useful as a debug tool, allowing the user application access to the hardware.

A detailed document, with operating instructions, can be found in the subproject’s directory (x270_CE6\Subprojects\SysConf)

# GDIRotation Utility

This is a command line utility, which can perform instant GDI rotation:

Syntax:

GDIRotation.exe angle

Where angle can be 0,90,180 or 270

The sources of GDIRotation.exe are located in C:\WINCE600\OSDesigns\x270_CE6\Subprojects\Rotation\GDIRotation\

and are available with a BSP package.

# AutoLaunch Utility

Syntax: AutoLaunch.exe

The AutoLaunch utility extends the functionality of the “HKEY_LOCAL_MACHINE\Init” registry key by requiring the network stack to be configured with an IP address, before launching additional applications.  It also allows each application to have its own configurable delay before being launched.

The AutoLaunch monitors the following registry key:

[HKEY_LOCAL_MACHINE\Startup]

“Process0” =”Application.exe param1 param2”

“Process0Delay”=dword:delay ;in milliseconds

Used to auto-launch the “CommanClien2.exe”, the CoreCon connection server.

# Notes

- To write an application in VS2005, you will need an appropriate SDK, available from CompuLab's website.